ENVIRONMENTAL

##### HEALTH & SAFETY

MANUAL

FOR

CONTRACTORS

ISSUED:  October 1996

Updated: April 2001

October 2002

November 2003

September 2004

BRIGGS & STRATTON CORPORATION

AND ITS SUBSIDIARIES

ENVIRONMENTAL, HEALTH & SAFETY MANUAL

FOR CONTRACTORS

SCOPE

This EHS Manual has been prepared by Briggs & Stratton Corporation for use by Contractors, their employees, subcontractors, and agents and any other person NOT employed by Briggs & Stratton who is working on property owned and/or operated by Briggs & Stratton Corporation or its subsidiaries (hereinafter referred to jointly as Briggs & Stratton).

For purposes of this Manual, the term “Contractor” includes the employees, subcontractors and agents of a Contractor and is anyone who is not, or whose employees are not, employees of Briggs & Stratton, including service company employees.  The term “service company employees” includes employees of a public utility  (telephone, electric, gas, etc.), oxygen and acetylene service, food service, and other companies similarly engaged.

Should Briggs & Stratton find Contractors, areas of work, or individuals in non-compliance with any Federal, State, or Local requirements, Briggs & Stratton shall have the authority to order immediate correction and cessation of the non-compliance.  All costs of abatement shall be borne by the Contractor deemed responsible by Briggs & Stratton. The Contractor shall be responsible for any liability, any delay to the project, and/or other performance requirements.  If the Contractor fails to make the necessary changes they may be removed from the job, and may be permanently removed from Briggs & Stratton’s Approved Contractor’s list.

SECTION 1

Overview

1.1  	Pre-project Meeting

The Contractor shall meet with its employees, subcontractors and agents before starting any work to review the requirements and procedures included in this EHS Manual.

1.2	Reporting of Incident or Injury

If there is an injury requiring medical treatment or property damage, the Contractor shall immediately notify the Safety Dept., plant Nurse, and/or Security    A written report of the event shall be submitted to the Safety Dept. within 24 hours.

1.3	Site Inspections

The Contractor’s Project Leader shall inspect the project and area of work on a daily basis to ensure that requirements and procedures of the EHS Manual.

1.4 	Signs and Safeguards

Contractors shall provide and properly maintain warning signs and lights, barricades, railings and other safeguards for the protection of workers and others on, about, or adjacent to the work, as required by the conditions and progress of the work.

1.5	Personal Protective Equipment

Contractors shall provide ANSI approved personal protective equipment (PPE). Safety glasses with side shields and hearing protection are required to be worn at all times. Steel-toed safety shoes while not required by Briggs & Stratton are highly recommended. The Contractor is responsible for providing PPE suitable to adequately protect from inhalation, ingestion, or dermal contact hazards associated with any material(s) used/activities conducted on Briggs & Stratton property. Briggs & Stratton reserves the authority to evaluate the hazards associated with any material used/activity conducted on Briggs & Stratton property to evaluate the selected PPE for adequacy as employed to eliminate or reduce the hazard in accordance with industry standard(s).

SECTION 2

Equipment

It is the responsibility of the Contractor to provide, maintain, inspect, and ensure that all necessary equipment, including PPE, is properly utilized or worn for the duration of the work.  All Contractor equipment shall be clearly marked as belonging to the owner prior to being brought onto Briggs & Stratton property.  Briggs & Stratton reserves the right to prohibit the use of any equipment, methods, or practices, which do not conform to industry standards.  All Contractor owned equipment is to be removed from Briggs & Stratton property immediately upon completion of the work in compliance with Briggs & Stratton security procedures.

SECTION 3

General Safety Rules

The following General Safety Rules shall be observed at all times while on Briggs & Stratton property.

3.1	Housekeeping

A good standard of housekeeping shall be maintained at all times.  Loose equipment, tools material, etc. shall be cleared from overhead areas before leaving each day.  Protruding nails in boards shall be removed or bent flush with the surface.  Aisle ways are to be kept clear.  Scrap shall be removed from the work area daily or more often if necessary.  Oily and solvent soaked rags may be a fire hazard and shall be deposited in the proper containers with self closing lids daily and upon completion of the work.  The Contractor shall leave his/her work area “broom clean” at the end of each workday.

3.2	Emergency & Hazard Reporting

The Contractor shall immediately report any emergency to Briggs & Stratton’s Security Department by dialing 4911 or the alternative number provided by Briggs & Stratton for the specific work site.  Any hazardous situations that are not immediately dangerous to life or health should be reported to the Contractor’s Supervisor and/or Briggs & Stratton’s Safety Department.

Briggs & Stratton may, at its discretion, request that the Contractor attend a training class designed to review site specific emergency procedures.

3.3	Overhead Work

Briggs & Stratton’s Corporate Plant Engineering and its Safety Department shall be consulted before working above areas with people working below.  When overhead work is in progress, the floor area below must be barricaded, roped off and posted with a sign to read “DANGER, KEEP CLEAR, PERSONNEL WORKING OVERHEAD.”

3.4	Overhead Loads

Suspended loads shall not be left unattended or transported over any personnel.

3.5	Load Capacities

Cables, slings, ropes, chains or any below the hook lift device shall be inspected for wear or defects prior to use.  Load capacities shall be noted and observed at all times.

3.6	Purging

Lines or containers that have been used for combustible materials shall be purged prior to the performance of any work.  The Contractor shall review the proposed work and obtain permission from Briggs & Stratton’s Corporate Plant Engineering, Facility Manager, and/or a site specific designated Department prior to beginning any work with these lines or containers.

3.7	Flash Shielding

When arc welding is to be done in the vicinity of other personnel, an approved shield shall be used.  Adequate ventilation shall be assured at all times.  The Contractor shall provide shields and ventilators.

3.8	Scaffolding

All planks and scaffolds overhead shall be secured and in compliance with OSHA 29CFR-1910.28.  Loose equipment shall not be left overhead where it may fall and injure someone.

3.9	Overhead Power Lines

No work may be done within fifty (50) feet of any overhead power line without first obtaining permission from Briggs & Stratton’s Corporate Plant Engineering or the Facility Manager.  The movement of mobile equipment around overhead electrical power lines shall be in accordance with Electrical Work Practice Standards and 29CFR 1910.331 - .335.

3.10	Tool Belts

When working overhead tools shall not be carried in pockets.  It is the responsibility of each Contractor to ensure that proper tool belts are being worn when working overhead.

3.11	Substance Abuse/Smoking Areas

Alcoholic beverages and controlled substances are strictly prohibited on Briggs & Stratton property.  Failure to comply shall result in the immediate and permanent expulsion of the individual and his/her employer.

Smoking is prohibited in all areas except the designated break areas and the cafeteria.

No loud, profane, lewd, or abusive language of any kind is allowed.

3.12	Floor and Roof Openings

All floor and roof openings shall be guarded to prevent personnel, stock, scrap or tools form falling through the opening as stated in OSHA 29CFR-1910.23.  A standard guardrail is a 42-inch high barricade with a mid rail and a four-inch high toe board.

3.13	Briggs & Stratton Equipment Use

Contractors should use only their equipment to complete the work.  In the event a Contractor requests permission to use Briggs & Stratton equipment such as forklifts, boom lifts, or scissors lifts the Contractor shall:

- Obtain approval from Briggs & Stratton’s Corporate Plant Engineering or Facility Manager

- Provide written documentation of all required training necessary to operate the specific equipment as required under 29 CFR 1910.

- Follow the safety guidelines for the use of the equipment.

- Return all equipment to its home location clean.

- Reimburse Briggs & Stratton for any damage to Briggs & Stratton property and/or equipment due to Contractor negligence.

3.14	Waste Disposal

Draining of gasoline, fuel oil, motor oil, flammable liquids, or other hazardous materials onto the ground, into an open sewer or any other non-approved location is prohibited.  All waste shall be disposed of in accordance with Briggs & Stratton policies and procedures and government regulations.  Briggs & Stratton’s Corporate Environmental Department or Site Specific Environmental Engineer shall be contacted to coordinate the disposal of any regulated substance and/or waste.

The Contractor agrees to properly dispose of all waste materials generated from the work performed on Briggs & Stratton property. Briggs & Stratton retains the right to request information concerning the intended disposal methods, sites to which the waste will be sent, and similar information. If at any time the disposal/recycling site, and/or disposal method is found to be unacceptable to Briggs & Stratton the Contractor shall make other arrangements, which are acceptable to Briggs & Stratton’s Corporate Environmental Compliance Department or Plant Environmental Engineer.

In addition, Briggs & Stratton reserve the right to request verification that waste or recyclable materials have been sent to the quoted disposal facility.

3.15	Material Blocking

Any large, heavy, rounded objects with a tendency to roll shall be chocked to prevent movement.

3.16	Traffic Signs

When operating mobile equipment on the property, Contractors shall obey traffic signs at all times.  Pedestrian traffic shall always be afforded consideration.

3.17	Vehicle Riders

No person other than the operator shall ride on mobile equipment unless the equipment is designed for more than one person and complies with OSHA standards.

3.18	Asbestos Removal

All Contractors removing asbestos must review the Briggs & Stratton “Conditions for Contractor Asbestos Removal” which will be provided as part of the quoting process.  The “Conditions for Asbestos Removal,” specifies that the Contractor shall be licensed and follow the applicable asbestos removal regulations.

SECTION 4

Specific Safety Procedures

4.1	Lockout/Tagout

To comply with OSHA 29CFR-1910.147, “The Control of Hazardous Energy” (Lockout/Tag out), and Briggs & Stratton procedures, each Contractor shall inform Corporate Plant Engineering and the Safety Department of their lockout procedure and in turn, be informed of Briggs & Stratton’s site lockout procedure.  Briggs & Stratton’s Safety Department shall review the Contractor’s lockout procedure to ensure completeness and compatibility with the Briggs & Stratton lockout program.  All tags shall provide the name of the Contractor and an emergency phone number.  Tags must be used at all times when lockout is used.

Contractor site management shall at all times enforce an energy lockout program for Contractor and Sub-Contractor employees when working on or near machinery or equipment where the unexpected movement or release of stored energy could endanger the Contractor or Briggs & Stratton personnel.

4.2	Confined Space

If work is to be completed within a confined space, the Contractor shall:

- Comply with OSHA 29CFR-1910.146

- Be aware of and have a copy of the Briggs & Stratton Confined Space Entry Program.

- Have a written procedure, which meets or exceeds the Briggs & Stratton program and submit a copy to Briggs & Stratton’s Safety Department at least one week prior to entry.

- Provide training appropriate for the role each person will perform during the job.  (Written verification required.)

- Ensure a written entry permit procedure is utilized prior to and during work in confined space.  A copy of the completed Contractor Entry Permit shall be submitted to the Briggs & Stratton Safety Department and retained for at least one year.

- Obtain access to the confined space prior to beginning work to allow the development of a rescue plan.

- Identify potentially hazardous materials and/or conditions prior to the start of work.

- Have its own equipment readily available to safely complete the work assignment.  This shall include rescue equipment, communications equipment, air monitoring equipment, which measures oxygen content, lower explosive limit, and hydrogen sulfide, etc.

- Report to the Briggs & Stratton Safety Department if any problems or unusual conditions are encountered while performing entry/exit operations.  For example:  Hazardous atmosphere, equipment malfunction, etc.

4.3	Electrical

All site electrical work, temporary or permanent, shall be in compliance at all times with the National Electrical Code (NEC).  Only “Qualified Persons” as that term is used in the NEC may complete electrical work.

Contractors working on electrical circuits and equipment must meet and follow the provisions of NFPA 70E as it relates to the “Qualified Worker” and wear all personal protective equipment in accordance with the standard. Contractors failing to abide to B&S safety policy or policies relating specifically to NFPA 70E will be immediately removed from B&S premises.

.4.4	Powered Industrial Trucks

Fork lift trucks shall be in good working order, equipped with an overhead guard and all necessary safety devices as in compliance with OSHA 29CFR-1910.178.  Trucks shall be of sufficient capacity to safely perform the assigned tasks.

All operators must be trained and licensed to operate vehicles.

Operators of all powered trucks shall stop and sound their horn at all doorways and intersections.  The raising of personnel with the forks of a truck is prohibited except when a properly engineered personnel platform with rails is secured to the truck.

4.5	Aerial Work Platform

Contractors shall comply with OSHA 29CFR-1910.67. Contractors who either direct or authorize the use of aerial work platforms shall ensure that all aerial work platform operators have been trained in accordance with the manufacturer’s operation manual before operating the aerial platforms.  Boom and/or scissors lift type aerial platforms are also included in this requirement.  Each Contractor site manager shall certify to Briggs & Stratton’s Corporate Plant Engineering and/or the Safety Department that all employees who are expected to use such equipment have been trained and licensed to operate said equipment. They must also certify that the equipment is in proper working order.

Guardrail gates on scissor lifts must be closed at all times. A full body harness and lanyard must be worn at all times while operating a boom (bucket) lift. The body harness lanyard must be secured to the lift or building structure.

4.6	Compressed Gas Cylinders

All compressed gas cylinders shall be stored in a ventilated area located away from vehicle traffic and free from falling materials.  Full cylinders shall be stored separately from empty cylinders.  Cylinders shall be fastened securely in an upright position at all times and stored with the valves closed and caps in place.  Oxygen and acetylene cylinders shall be separated by a non-combustible partition or stored twenty feet apart.  All vehicles used to transport cylinders shall be designed in such a manner that the cylinder is securely fastened in an upright position.  When cylinders are empty, they are to be marked as such and taken to the empty storage area immediately.

SECTION 5

Fire Regulations

Contractors shall take all necessary precautions to eliminate all possible fire hazards and to prevent damage to any buildings, materials, equipment and all other property.

  - Burning and Welding (Hot Work Permit)

A Hot Work Permit is required for any temporary operation involving open flames or producing heat and/or sparks.  This includes but is not limited to: brazing, cutting, grinding, soldering, torch applied roofing and welding. The permit needs to be obtained from Briggs & Stratton’s Project Engineer, Maintenance or Safety Department prior to the start up of any such work.

Contractors shall not weld, flame cut, or use flame, arcs, or sparking devices without:

a). Adequate protection and shielding, particularly at the point of operation for Contractor as well as Briggs & Stratton employees.

b). The permission of the Briggs & Stratton Facility Manager or Safety Department.

All combustible or flammable material shall be removed from the immediate work area. If removal is not possible, all flammable or combustible materials shall be protected with suitable non-combustion shields to prevent sparks, flames, or hot metal from reaching the flammable or combustible materials. Contractors shall provide the necessary personnel and fire fighting equipment to effectively control fires resulting from welding, flame cutting, or other operations involving the use of flam, arc, or sparking device. The work area shall be constantly monitored during work and up to 4 hours after the work is completed.

5.3	Flammable Liquids Storage

Only a one-day supply of flammable liquids such as oil, gasoline, paint or paint solvents may be brought onto the work site.  All flammable liquids having a flash point below 1100 F. brought into any building shall be contained in Underwriters Laboratory approved and properly labeled safety cans.

5.4	Extinguishers

Contractors shall provide adequate fire extinguishers in working order and properly filled.  These extinguishers shall be checked by the Contractor each morning before starting work and are required on all welding jobs.

5.5	Grounding/Bonding

All flammable liquids containers shall be properly grounded and bonded during the transfer of flammable liquids from one container to another.

5.6	Ignition Sources

The use of extension lights, electric drills, grinding wheels, or any spark producing device shall not be permitted in any area where flammable liquids are stored or used.

5.7	Fire Reporting

All fires shall be reported to Security, giving specific location and approximate size of fire by dialing 4911 or the alternative number provided by Briggs & Stratton for the specific work site at the nearest plant phone.

SECTION 6

Hazard Communication

A hazardous material is defined to be any biological or chemical agent as identified by the requirements of OSHA Hazard Communication Standard 29CFR 1910.1200.

6.1	Hazard Communication Training

The Contractor is responsible for providing all training and instruction necessary to comply with OSHA Hazard Communication Standard 29CFR 1910.1200 with respect to all hazardous materials they will work with on Briggs & Stratton property including emergency procedures as required by the State or Federal Occupational Safety and Health Act.

6.2	Chemical Approval

All hazardous materials to be used by Contractors on Briggs & Stratton property must by approved by Briggs & Stratton prior to being brought on Briggs & Stratton property.  The Briggs & Stratton Product Approval process will be followed to obtain such approval.  The Product Approval Forms can be obtained from Briggs & Stratton’s Safety Department.

Briggs & Stratton retains the right to review materials used by any Contractor at any time while on Briggs & Stratton property. If at any time Briggs & Stratton determines that a material is incompatible with any equipment, machinery, process, or treatment system, use of that material shall be discontinued until such time as a suitable replacement is approved by Briggs & Stratton.

6.3	Labeling

All containers of hazardous material brought onto Briggs & Stratton property by the Contractor shall be properly labeled.  If hazardous material is transferred to a secondary container, the proper label shall be affixed to the new receiving vessel.  The labels shall include information identifying the hazardous material, provide safe handling instructions and the availability of the Material Safety Data Sheet.

6.4	Material Safety Data Sheets (MSDS)

MSDS for all hazardous materials used or produced by the Contractor while on Briggs & Stratton property shall be in the possession of the Contractor at all times while on site, and made available upon request to affected Briggs & Stratton employees for their examination.  The MSDS shall disclose information about the hazards, possible health effects and protective measures associated with the use of the hazardous material.

ACKNOWLEDGMENT

The Contractor listed below has received and reviewed a copy of Briggs & Stratton Corporation’s “Environmental, Health and Safety Manual for Contractors” and fully understands and agrees that the policies and procedures contained therein shall be complied with by the Contractor, its employees, subcontractors and agents.

In accordance with the requirements of OSHA Hazard Communication Standard 29CFR 1910.1200, Briggs & Stratton maintains files of Material Safety Data Sheets (MSDS) on hazardous materials for each of its facilities.  If anyone should have a question regarding any hazardous material s/he may come in contact with, an MSDS on that material should be reviewed.  MSDS’s can be obtained from Briggs & Stratton’s Safety Department upon request.

Acknowledged

The Contractor, its employees, subcontractors and agents shall comply with all Federal, State, and Local regulations and the policies and procedures described in Briggs & Stratton’s EHS Manual applicable to the work performed for Briggs & Stratton.

Contractor’s Name:  _________________________________

Contractor Representative Name:  _________________________________

Title:  _________________________________

Signature:  _________________________________

Date:  _________________________________

1.  FORWARD ORIGINAL COPY TO LEGAL DEPT.

2.  MAINTAIN A COPY IN CORPORATE PLANT ENGINEERING

3.  MAINTAIN COPY IN ENVIRONMENTAL DEPT.

4.  MAINTAIN A COPY IN SAFETY DEPT.