# Company Wide Equipment Standards

# And RFQ Packet

BRIGGS & STRATTON CORPORATION

AND ITS SUBSIDIARIES

EQUIPMENT PURCHASE AGREEMENT

ATTACHMENT “B”

Revision Date

August 1, 2005

CONFIDENTIAL/PROPRIETARY INFORMATION

PROPERTY OF BRIGGS & STRATTON CORP.

Project Name:

Attachments: (check box if attached)

Part Prints

Machine & Process concept drawings

Copy of signed Confidentiality Agreement

Standard Terms & Conditions

Health & Safety Manual

Date Equipment Manual Sent:

Date RFQ Packet Due:

Date RFQ Packet Received:

DOCUMENT CONTROLLED BY: Corporate Technical Services Manufacturing Engineering Manager

REQUESTS FOR CHANGES:  Requests for modifications to this document need to be made to the CTS ME Manager and reviewed and approved by the Equipment Standards Review Committee.

#### Table of Contents

Project Description

- General

- Quotation Instructions

- Terms & Conditions

- Approval Drawings

- Documentation & Drawings

- Safety & Ergonomics

- Electrical

- Hydraulic Systems

- Pneumatic Systems

- Lubrication Systems

- Mechanical Design

- Tooling

- Equipment Acceptance Testing

- Standards Deviation Request

- Company Profile Sheet

### Appendices

Appendix – A  Component Specifications

Appendix – B   Lubrication List

Appendix – C   Recommended Spare Parts List Form

Appendix – D  Terms and Conditions of Equipment Purchase & Schedule A

Appendix – E   Vendor Instructions for Tool Drawings

Appendix – F   Revision level details

Company-Wide Equipment Standards & RFQ Packet Rev. August 1, 2005

Confidential and Proprietary Information      Property of Briggs & Stratton Corp

### Page 2 of 78

### Project Description

Equipment/Process Description:

B&S Project Leader:

Phone #      				Fax #

Pager #      					Cell Phone #

E-Mail:

THIS B&S COMPANY-WIDE EQUIPMENT STANDARD IS LOANED TO:

Company:

Address:

Component Information:  (To be completed by B&S Project Leader)

Part Name:

Part Number:

Revision Level:

Print Number(s):

ECO/ECR #:

MCR/MCO #:

Equipment Information:  (To be completed by B&S Project Leader)

Required Cycle Time:

Required Efficiency:

Required Changeover Time:

Order Date:

Delivery Date:

Production Date:

Method of Shipment:

Ship to:       FOB:

Special Requests:

Company-Wide Equipment Standards & RFQ Packet Rev. August 1, 2005

Confidential and Proprietary Information      Property of Briggs & Stratton Corp

Page 3 of 78

Payment Schedule:

1) Purchase Order:      %

2) Drawing Approval:      %

3) Acceptance At Company:      %

4) Acceptance at B&S and receipt of all drawings, documentation:      %

Company-Wide Equipment Standards & RFQ Packet Rev. August 1, 2005

Confidential and Proprietary Information      Property of Briggs & Stratton Corp

Page 4 of 78

I.   GENERAL

This manual describes and defines the standard operating requirements, standards, and specifications for the equipment, parts and accessories to be used by Briggs & Stratton Corporation and its subsidiaries.  "COMPANY" shall be used throughout this specification to mean the equipment supplier or manufacturer.  "B&S" means Briggs & Stratton Corporation and its subsidiaries.

COMPANY will review these standards at the time of equipment quotation and base pricing and terms on strict adherence to these standards.  Where strict adherence to any detail items of B&S standards would result in increased cost without comparable benefit, COMPANY shall indicate such items to B&S for clarification.  COMPANY may not deviate from B&S standards without specific written approval from B&S designated Project Leader.

Any secondary supplier used by the COMPANY shall adhere to these same standards.  The COMPANY is responsible for ensuring that any secondary suppliers to follow B&S requirements, standards, and specifications.  The COMPANY will notify B&S if any secondary suppliers are to be utilized.

The specifications and enclosures herewith are submitted in strict confidence for the sole purpose of your evaluation in considering a proposal.  Information contained may include trade secrets/methods and such information shall be maintained in confidence and secrecy, and not disclosed to any other person or persons without prior knowledge and express written consent of B&S.

This Company-Wide Equipment Standard and RFQ Packet is loaned to the COMPANY for the sole purpose of preparing and submitting a quotation for the machine in accordance with the equipment standards.  If the COMPANY desires additional copies or additional information relating to the machine or project, then the COMPANY shall request such copies or information from the B&S Project Engineer.

This document and the TERMS & CONDITIONS OF EQUIPMENT PURCHASE take precedence over any conflicting COMPANY stated specifications, terms and conditions and shall control in any contract between COMPANY and B&S.

The COMPANY shall provide a certificate of insurance to B&S prior to any work on B&S premises.  The certificate shall show coverage as required in the TERMS & CONDITIONS OF EQUIPMENT PURCHASE.

B&S reserves the right to access COMPANY’s plant to inspect work in process.  Unless otherwise agreed upon, inspections will be conducted during normal business hours.

Company-Wide Equipment Standards & RFQ Packet Rev. August 1, 2005

Confidential and Proprietary Information      Property of Briggs & Stratton Corp

Page 5 of 78

II.   QUOTATION INSTRUCTIONS

- All COMPANY quotations shall include a copy of these standards completely filled out and signed by an authorized COMPANY representative.  All boxes on the left hand side of form are to be checked either yes or no by an authorized COMPANY representative at the time of quotation.  A yes check acknowledges that the specification is acceptable to COMPANY.  A no check is a request for deviation and indicates COMPANY believes that:

- Adherence to the specifications would result in increased cost without comparable benefit.

- Specification is not applicable to this particular project or equipment. (e.g. coolant system specifications do not apply to an assembly equipment project. )

- Adherence to specification would result in significant delay of equipment delivery.

- Adherence to specification would adversely affect equipment performance.

Each no answer shall be explained and documented in Section XIV Standards Deviation Request.  In no case may the COMPANY proceed without written authorization from the B&S Project Leader.  (B&S Project Leader to sign initials and note approval status by each exception.)

2.	The quotation price shall be firm for a minimum of 60 days and shall be in U.S. dollars.

3.	The quotation is to include an itemized break-down of the following costs:

a.	Basic machine, equipment, and/or system.

b.	Control system: electrical, hydraulic, and pneumatic, if applicable.

c.	Tools and tool holders, if applicable.

- Coolant tank/pump system, if applicable.

- Pallets and fixtures, if applicable.

- Parts handling system, if applicable.

- Additional runoff costs, if applicable.

- Shipping.

- Optional equipment (e.g. mist collection, etc)

- Any custom duties, import taxes, licenses, permits, and custom brokerage.

- Estimated spare parts costs.

- Estimated size and weight.

- Total cost with and without optional features.

- Any other items requested by the B&S Project Leader.

Company-Wide Equipment Standards & RFQ Packet Rev. August 1, 2005

Confidential and Proprietary Information      Property of Briggs & Stratton Corp

Page 6 of 78

QUOTATION INSTRUCTIONS - Continued

4.	The following information shall also be included in all quotations:

- Returned B&S "Equipment Standards"

b.	Proposal layouts/concept drawings on all special machines, equipment or systems.

c.	Cycle time breakdown including:

- Guaranteed maximum equipment cycle time.

- Estimated machining rapid and feed times

- Estimated manual load/unload time

d.	Manufacturer's name, type and part number for major components. (Spindles, slides, etc.)

e.	Delivery time from receipt of order.

f.	List of all software used and version number.

g.	Any recommended training to operate and maintain the equipment.

h.	Rough estimate of spare part costs.

i.	Any other information requested by the B&S Project Leader.

- The COMPANY will provide the on-site services of an experienced, qualified individual to supervise installation and to train B&S personnel in the operation and maintenance of the equipment.  The cost of this service shall be included in the total bid price.  The installation supervisor shall be available until the equipment has been ACCEPTED by B&S.

III. TERMS & CONDITIONS

YES	 NO

1.      	The COMPANY understands and accepts the terms and conditions set forth in the B&S TERMS & CONDITIONS OF EQUIPMENT PURCHASE in this manual  (Appendix D).

Company-Wide Equipment Standards & RFQ Packet Rev. August 1, 2005

Confidential and Proprietary Information      Property of Briggs & Stratton Corp

Page 7 of 78

### IV.   APPROVAL DRAWINGS

The responsibility for the capability of the design and function of the equipment lies solely with the COMPANY unless otherwise agreed upon in writing and in conjunction with specific changes requested by B&S.   Required approval drawings are as follows:

YES	 NO

1.	  	General layout and floor plan top view drawn in full scale showing:

a. Major dimensions of equipment.

b. Support equipment in accurate location (filters, hydraulic tanks, electrical cabinets, loaders, etc.)

c. All access doors and their swing.

d. Operator load/unload point.

e. Connection points and utility requirements including:

- Voltage and amperage for each electrical disconnect.

- Pressure, volume and inlet pipe size requirements for all pneumatic, coolant, steam or other systems.

2.	       	Elevation drawing showing all major height dimensions such as operator load/unload heights, equipment clearance heights, etc.

3.		    Fixture layouts.

4.		  	Tool layouts documenting feed rate, feed distance, spindle speeds, rapid-traverse speed, rapid-traverse distances, tooling type, manufacturer, material, etc.

5.	  	Ergonomic layouts of manual workstations.

6.	  	Electrical hardware and programming schematic drawings as outlined in section VI, "Electrical" of these standards.

7.	  	Hydraulic schematic drawings as outlined in section VII, "Hydraulics" of these standards.

8.	  	Pneumatic schematic drawings as outlined in section VIII, "Pneumatics" of these standards.

9.	  	Lubrication schematic drawings as outlined in section IX, "Lubrication Systems" of these standards.

10.	  	Detailed electrical, mechanical, and purchased component Bill of Materials.

V.   DOCUMENTATION AND DRAWINGS

Within two weeks of receipt of the Purchase Order, COMPANY shall provide B&S with a project timeline outlining the following project target dates:  design start, design completion, build start, build completion, equipment acceptance, and shipment.  In the event of any changes to the schedule, COMPANY shall notify B&S immediately and provide an updated timeline.

COMPANY shall supply complete documentation as listed below before equipment ACCEPTANCE and payment.

YES 	 NO

1.	  	COMPANY shall supply B&S with a complete set of original detail drawings for all specially designed equipment.  Preliminary drawings are acceptable upon delivery.  All final drawings are required (including all changes of how machine was actually built) no more than 30 days after machine is accepted at B&S.  All drawings shall be in electronic format, shall use B&S borders and be supplied on computer disk in AutoCAD drawing format.  (Project leader will consider special requests for dxf, IGES, or 3D CAD formats.)  All drawings shall adhere to B&S "Vendor Instructions for Drawings” in Appendix E and use the B&S drawing numbering system with a set of drawing numbers supplied by the B&S Project Leader.

2.	  	COMPANY is to supply B&S two (2) hard copies of operating and maintenance manuals, including all available manufacturers manuals for purchased components as well as one copy in electronic format (check with Project Leader for acceptable file types).  These manuals shall be included with the machine delivery, and shall include the following information:

a. COMPANY's name, telephone number and project identification number per section XV “Company Profile Sheet” of these standards.

b. Basic equipment start-up and operating instructions.

c. Recommended preventative maintenance program.

d. A complete list of consumable items (filters, tools, oils, etc.)

e. Maintenance instructions.

f. A complete parts list with all purchased parts identified by the original manufacturers’ names and part numbers.

g. Name and telephone number of any company supplying major components (spindles, slides, coolant system, etc.)

h. Recommended lubricants. (ref: Appendix B – Lubrication List)

i.  COMPANY shall supply B&S with the “KEY PROCESS VARIABLES” within two weeks of equipment acceptance at COMPANY facility.

3.	  	COMPANY is to provide a recommended spare parts list using the form in Appendix C before shipment.

DOCUMENTATION AND DRAWINGS - Continued

YES  NO

4.		  	Complete electrical schematic and program documentation as outlined per    section  VI, "Electrical" of these standards.

5.		  	 Complete hydraulic schematics as outlined per section VIII, "Hydraulics" of these standards.

6. 	  	Complete pneumatic schematics as outlined per section IX, "Pneumatics" of these standards.

7. 	   	Complete lubrication schematics as outlined per section X, "Lubrication Systems" of these standards.

8. 	  	B&S reserves the right to reproduce, in total or in part, all documents furnished by COMPANY, as necessary for maintaining, installing, or operating equipment. Therefore, all documents shall be of a quality, which will permit making readable, usable copies by a common reproduction process such as photograph, electrostatic copying, or blueprint.

VI.   SAFETY AND ERGONOMICS

Safety

YES	 NO

1.	  		All equipment shall conform to Joint Industrial Council (J.I.C.) standards, National Electrical Code and any applicable state administrative codes for electrical, hydraulic, pneumatic, and mechanical systems.  Where these standards overlap or conflict with this document, the more stringent shall take precedence.

2.	  		All equipment shall conform to the Occupational Safety and Health Act of 1970 (OSHA) and amendments thereto:  (e.g., noise levels, guards, etc.).  All equipment shall be furnished with sufficient safety devices of the latest approved type, including guarding of all hazardous moving parts.

3.	  	 	Where applicable standards have been set, all equipment, parts and accessories shall be manufactured and/or fabricated and installed in strict compliance with, (and if applicable, bear the seal of) UL, NFPA, NEMA, ANSI, ASME, NSF and FM-Global Insurance standards.  Equipment accessories and installation shall comply with all rules, regulations, codes and interpretations of same, and of any governmental or regulatory agency having jurisdiction over this installation.

4.	  		All unnecessary sharp edges and corners shall be removed.

SAFETY AND ERGONOMICS – Continued

YES  NO

5.	  	Equipment shall have adequate guarding to protect operators and other personnel from pinch-points and moving parts.  All moving power transmission mechanisms such as belts, pulleys, shafts, chains, and couplings shall be fully guarded.  Guarding shall be interlocked that prevents machine from operating in automatic when covers are removed or doors are open.

6.	  	Where light curtains are used, equipment mechanical and electrical systems shall respond fast enough to light curtain signals to stop equipment before injury can occur.

7.		  	Equipment controls shall be properly labeled as to their function and located for easy operator access at a safe distance from equipment point of operation.

8.		  	All new equipment shall be ergonomically designed for standardized work heights, reaches, control locations and minimal work movements. Reference Mechanical Design, Section XI, Item #1.

9.		  	Equipment shall have lockout capability for all power sources.  If feasible, single source lockout capability shall be utilized.

10. 	  	Equipment that produces mist, dust, vapors or fumes shall be properly ducted, filtered, and vented.  All mist, vapor or fume producing equipment shall identify the operator work zone and maintain a net increase of particulate levels below 0.25 mg/m3 within that zone when equipment is in full operation.  Additionally, non-work zone areas shall be held within .5 mg/m3 or as approved by B&S environmental requirements.

11.  	  	Piping that will sweat or cause burns if touched shall be insulated.

12.  		     Guards or collars shall be used to reduce the risk of accidental actuation.  Guards on top of the hand control are preferred over collars.  Where accidental actuation does not present a special hazard because of the type of machine, the speed of the machine, or other guarding methods, collars should not be used since they lead to awkward wrist positions.

13.	         	Enclosures shall confine process fluids within the machine.

14. 		    	No coolants, oils, or chips are permitted on the floor, unless machine is designed with a wet floor.

SAFETY AND ERGONOMICS - Continued

###### Ergonomics

YES   NO

1.	  	Heights - Unload/load height of equipment(s) 36-44" for a standing operator.  Work heights for sitting operator should be 26-34”.

Reaches - Station layout should allow for easy reaches.  Avoid holding arms in an extended position.  Place primary items in the first 10” of space and secondary items in the first 20 inches.  Spaces beyond 20 inches should only be used for storage.

Hand Controls – Hand controls should be located as close to work height as possible.  Controls shall be located no more than eleven inches above nor more than three inches below working height.  Controls should be oriented at approximately the same angle as the forearm during operation (to minimize bending of wrist).  Any hand control located within three inches of working height should be horizontal.  The horizontal separation between controls should be between 20-36”, and symmetrically positioned in relation to the working position of the operator.  Check with B&S Project Leader for specific requirements.

2.	  	The workspace shall be at least be 28”x 40”.

3.	  	The machine operator shall have minimum 4” vertical and minimum 6” horizontal clearance for foot placement.

4.	    	The machine operator shall have a minimum of 81” height clearance from the operator’s foot location at the load station.

5.	      Shelf heights shall be 48” or less

6.	     Lift tables shall be available to adjust load and unload heights.

7.	     Lift/tilt tables shall be available to adjust part storage box angle presentation.

8.	     Part containers shall be designed to limit weight to less than 40 lbs. (based on part size).

9.	     Equipment shall be provided to keep manual lifts 27”- 48” above working platform.

10. 	     Handle lengths on hand tools shall not be greater than 4.5” for bare hands and

shall be greater than 5” for gloved hands.

SAFETY AND ERGONOMICS  - Continued

11.	     The diameters of hand tool handles (screwdriver type) shall be 1.25”-1.75” for pinch    grips.

12.	     The diameters of hand tool handles (pliers type) shall be 0.3”-0.6” for pinch grips.

13.	     Hand tool handles (pliers type) shall have spans of 2.5”-3.5”.

14.	     Hand tool triggers shall be greater than 1” in length.

15. 	      Part transfer container handles shall be less than 1.5” in diameter.

16.	     Torque limiting devices shall be provided with:

      - Right angle tools with a torque greater than 177 in-lbs.

      - Pistol grip tools with a torque greater than 24 in-lbs.

      - Inline tools with a torque greater than 14 in-lbs.

17.	     Push buttons shall require less than 2 lbs. of force to activate (for non-cycle start operations).

18.	     Trigger forces shall be less than 6 lbs.

VII.   ELECTRICAL

1.	  	All equipment shall operate on 460 volts, 3 phase delta 60 cycle incoming power.  Any isolation transformers, step-down transformers, power supplies, and/or power conditioners required for proper equipment operation shall be provided by COMPANY.

2.	  	All equipment wiring shall be copper and conform to the following color code:

RED - 120 VAC control wiring

BLACK - 120/240/480 VAC power wiring (DC motors shall have (+) and (-) marked on the respective wires.   All 240/480 VAC power wires shall be minimum size 12 AWG).

WHITE - neutral wires

GREEN - ground wires

BLUE - DC control wires - SHALL BE SEPARATED FROM VAC WIRING

YELLOW - interlocking wires between separate power sources

COMMUNICATION CABLE - shall run in its own wire way.

3.	  	All PLC I/O shall use the I/O point address as the wire number.

ELECTRICAL  - Continued

YES NO

4.	  	All wires more than six (6) inches long shall be numbered at each end.  Wires six inches or less in length need only be marked on one end.  Terminal blocks shall also be marked with the respective wire number.

5.	  	The main control panel shall have at least 20% free back panel space.

6.	  	Junction boxes larger than 8" x 10" shall use wire duct to assist in wire routing.  Terminal strips shall have a minimum of 1.5" clearance to the wire duct and 3" to the side of the junction box.  Junction boxes 8" x 10" and smaller shall have a sufficient number of terminal blocks to terminate all wires in that box.  All boxes shall have 10% extra terminals or two (2) extra, whichever is greater.

7.	  	All conduits shall have a minimum of 10% spare wires or two (2) spare wires, whichever is greater.  The maximum number of wires in any conduit shall not exceed National Electrical Code standards.

8.      	All systems shall be designed for fail-safe operation.  All "emergency stop" provisions shall prevent the equipment from any subsequent movement.

9.	  	All 115 VAC control systems shall have a grounded neutral.  Step-down transformers shall be fused on the primary and the secondary.  Equipment requiring a floating neutral control voltage needs special approval from B&S.

10.	  	All single phase and 3-phase transformers shall be externally mounted.

11. 	  	If the equipment uses programmable controllers, B&S shall approve the hardware configuration, in writing, before any is purchased.  The COMPANY shall provide two open I/O slots or 25% free I/O, whichever applies, and 25% free memory.  All input/output devices shall be tagged with their respective addresses.

12. 	  	A 120V GFCI protected duplex receptacle separately fused at 2 amps shall be required outside the panel with appropriate shielding for all equipment using programmable controllers.  In addition, remote access ports shall be required outside the panel for all equipment with programmable electrical devices.

13.	  	Any variable speed, variable frequency, servo and/or spindle drive systems shall be approved, in writing, by B&S before any hardware is purchased.  Special approval is required for stepper motors or stepper motor systems.

14. 	  	Electric motors 1/2 horsepower and larger shall operate on 460V, 3 phase, 60 cycle power.

ELECTRICAL  - Continued

YES NO

15. 	  	COMPANY shall provide any necessary heating/cooling equipment to keep components inside the electrical enclosure within their operating temperature limits based on an ambient temperature of 100° F.  Any cabinet coolers shall contain an approved non-CFC refrigerant.

16.	   	All electrical documentation shall be as follows:

a. 	Hardwire drawings supplied on AutoCAD Rev. 2000i or Mechanical in dwg format. (Special requests may be granted for dxf or IGES format.) B&S will supply an outline border and the required drawing numbering sequence.

b. 	All PLC programs shall contain descriptors on all program contacts and coils.

c. 	Programmable controller programs shall be documented in accordance with Rockwell Automation RSLogix.

NOTE: Any convertible software shall be pre-approved by the B&S Project Leader

d. 	All PLC and ACAD files shall be supplied to B&S on CD ROM or USB 2.0 flash media at time of delivery.

e.	Hard copies of the program and schematics shall be supplied to B&S at the time of equipment delivery.

17.	  	Controls shall be fused as follows:

a. 	All motors 480VAC, 3-phase shall be individually fused with 600V, LPJ low peak fuses with indicating finger-safe fuse holders.

b. 	All PLC outputs shall be individually fused.  Multiple station equipment shall have station input groups separately fused.

c. 	All main disconnects shall be fused (no circuit breakers).

18. 	  	All devices and plastic wire way mounted to back panels shall use screws into drilled and tapped holes.

19.	  	All operator interface panels shall be approved, in writing, before any hardware is purchased.

20.     	IEC type devices are permitted only with written approval from the B&S Project Leader.

ELECTRICAL  - Continued

YES NO

21.	  	All grounding shall be in accordance with Article 250 of the NEC or the local codes whichever is more stringent.  This includes, but is not necessarily limited to:

a. 	Bonding of multiple door enclosures to form a common ground.

b. 	Bonding of all conduits and raceways to all metal boxes and the main enclosure.

c. 	Proper grounding of all electronic equipment per manufacturer's instructions.

22. 	  	All terminal boxes and junction boxes shall be readily accessible by maintenance personnel.

23. 	  	Any enclosure used for operator interface (i.e. push-buttons, pilot lights) shall include terminal strips for wire termination before device.

24.  	  	All cord devices shall be terminated on terminal strips.

25. 	  	All input and output devices shall have an identifying tag attached on or near the device to simplify trouble-shooting and maintenance.  The tag shall list the device I/O address number and a short description consistent with controls documentation.

26. 	  	The B&S Project Leader shall approve the placement of electrical panel.

27. 	  	B&S "Component Specification List" for the specific plant shall be followed.  Any deviation will require written approval by the B&S Project Leader.

28. 	  	All equipment shall have an electrical cycle counter installed.

29. 	  	All valve solenoids shall be plug-in style, if available.  Coils on solenoids shall have indicating lights, if available.

30. 	  	Multi-station transfer-lines and dials shall have provisions for manual control of tool slides at each station.  The B&S Project Leader shall provide specific requirements.

31.		  	A control station shall be provided that allows for Auto and Manual control of the equipment. Auto control is the equipment cycle as defined by the B&S Project Leader. Manual control is defined as the ability to individually jog/start/stop/clamp/unclamp all components of the equipment.  Control interlocks shall be provided to prevent equipment damage from occurring in manual mode.

ELECTRICAL  - Continued

YES NO

32. 	  	If cabinet cooling is required the method of cooling shall be approved by the Project Leader and stated in the quotation.

33. 	  	The main electric panel shall be designed so the control power and the motor control power is separated (isolating the 480 VAC if possible).

34. 	  	 All fuse holders shall be finger-safe and indicating type.

35.	  	All exposed terminals shall be guarded.

36. 	    	All electrical components on equipment using coolant in the machining operations shall    have Viton seals and shall carry a rating of at least IP67 or better.

37.   	   	All spare wiring shall be terminated at both ends.

38.  	  	 Separate power supply is required if a vision system is used.

39. 	  	 Safety glass windows shall be installed in the panel doors to allow viewing of the PLC I/O status without opening the door and shall carry a rating of at least IP67 or better.

VIII.   HYDRAULIC SYSTEMS

YES NO

1.	  	COMPANY is responsible for ensuring proper circuit design and function.

2.	  	All hydraulic systems shall conform to the latest ANSI/(NFPA/JIC) T2.24.1 standard except where such conformation is in conflict with this document.  In any such conflict, this document shall have precedence.

3.	  	All pumps inlets will be equipped with a strainer-type inlet filter.  The filter shall be accessible without removal of the hydraulic pump.

4.	  	All systems shall have a return line filter equipped with a bypass check-valve and backpressure indicator gauge.  Filter nominal flow capacity shall be at least 1.5 times greater than the maximum flow rate in the return line.  Filters shall meet a minimum rating of ISO 17/14 or Beta (15µm - 25µm)>75.

5.	  	Systems are to be designed for the lowest feasible operating pressures not to exceed 1000-psi maximum.  Where space is a constraint clamping circuits may operate at up to 3500 psi if approved in writing by the B&S Project Leader.

HYDRAULIC SYSTEMS - Continued

YES NO

6.	  	All fixed displacement pump systems shall include an appropriate pressure unloading circuit.   Exceptions may be made for low pressure/low volume applications if approved in writing by the B&S Project Leader.

7.	  	All fluid lines shall be high quality seamless steel tubing or suitable hydraulic hose.  Tubing and hose shall be sized using a minimum safety factor of 4:1 (burst pressure/peak operating pressure).  Minimum acceptable tubing wall thickness is as follows:

TUBE O.D."	WALL THICKNESS (min.)

1/4	.028

5/16	.035

3/8	.035

1/2	.049

5/8	.065

3/4	.065

1	.095

1 1/4	.109

1 1/2	.134

8.	  	Inside diameter of lines shall be sized using the following maximum fluid velocity guidelines.

- Working lines -		15 feet/sec.

- Pump inlet line -		  4 feet/sec.

- Tank return lines -		10 feet/sec.

9.	  	All tubing runs shall be supported with cushioned mounting as follows:

LENGTH BETWEEN

TUBE O.D."	 SUPPORTS (Feet max.)

1/8 - 3/8	3 '

1/2 - 1	5 '

1 1/4 - up	7 '

10.	  	All tank return lines shall extend below the lowest anticipated fluid level and shall be isolated from the pump by at least one baffle.

HYDRAULIC SYSTEMS - Continued

YES NO

11.	  	Flareless fittings as approved in B&S "Component Specification List" or 37 degree flared 3 piece fittings are to be used for tubing connections.  Flareless fittings are preferred.  Threaded connectors, cylinder ports, valve ports, etc. shall be SAE straight thread if available.  NPT or NPTF connections are to be avoided.

12.	  	Pressure gauges shall be glycerin filled and be isolated from pressure spikes by a snubber or "push to test" valve.  Gauges shall be mounted as close to their respective pressure control device as possible.

13.	  	Reservoir tank capacity (gallons) shall be a minimum of 3 times pump output (gpm). A thermometer and fluid level sight gage shall be permanently mounted to the reservoir.

14.		    The B&S Project Leader shall approve and document the approval when using cartridge, proportional, and servo valves on the equipment.

15.	  	All valves, valve bodies, and manifolds shall be readily accessible for maintenance.

16.	  	Systems shall be designed so heat exchangers are not required and systems will operate within 45 degrees F of ambient temperature.

17.	  	Feed circuits for machining operations shall be isolated from all other circuits that may cause pressure fluctuations during the feed cycle.  Pressure and temperature compensated flow control valves shall be used for all machining feed applications.

18.	  	Rapid to feed over-ride control valves shall be single solenoid spring return type.  The valve shall be installed so that it is energized for rapid and will spring return to feed mode.

19.	  	Hydraulic documentation shall include copies of catalog information on all system components.  Any available maintenance instructions shall also be provided. Hydraulic bill of material shall list components by original manufacturers part numbers. This includes any components purchased as assemblies from a third party by the machine builder.

20.	  	The B&S "Component Specification List" for the B&S plant where the equipment is being installed shall be followed.  Any deviation requires written approval by the B&S Project Leader.

HYDRAULIC SYSTEMS - Continued

YES NO

21.	  	The Hydraulic system drawings will comply with B&S “Vendors Instructions for Drawings”.  No pencil drawings will be accepted.  Drawings shall be CAD generated in full-scale geometry and scaled during printing, with scale in title block reflecting printing scale.

22.	  	All valve solenoids shall be plug-in style if available.  Coils on solenoids shall have indicating lights if available.

23.	  	All valves shall have an identifying tag attached on or near the device to simplify trouble-shooting and maintenance.  The tags shall include the PLC I/O address and functional description.  All pressure controls and switches shall have an identifying tag showing proper pressure setting.

24.	  	Cylinders shall have SAE straight thread ports.  Integral cushions or deceleration circuits shall be used where required to minimize mechanical shock when moving heavy loads at high velocities.

25.	  	Accumulator circuits shall be designed to automatically discharge any pressurized fluid upon pump shutdown.

26.	  	Spool type directional valves shall be mounted with the spool axis horizontal.

27.	  	All reservoirs shall have a plate on the outside specifying the proper type oil to be used.  Both Mobil Oil and ISO specifications shall be included.

28.	  	Reference “Lubrication List” in Appendix B for a list of common hydraulic fluids used in each B&S location.  Any lubricate, hydraulic oil, or grease shall conform to the list for normal applications.  As new lubricants and hydraulic oils are required, the written approval of the B&S Project Leader is required so “Lubrication List” can be revised.

IX.   PNEUMATIC SYSTEMS

YES	NO

1.	  	All Pneumatic systems shall conform to the latest ANSI/(NFPA/JIC) T2.25.1. Standards except where such conformation is in conflict with this standard.  In any such conflict, this standard shall have precedence.

2.	  	Compressed air piping shall be galvanized schedule 40 pipe or seamless steel tubing.  Nylon tubing may be used between valves and actuators.

3.	  	Pneumatic documentation shall include copies of catalog information on all system components.  Pneumatic system drawings shall be to ANSI/(NFPA/JIC) standards.  The pneumatic bill of material shall list components by original manufacturers name and part numbers.

4.	  	The Pneumatic system drawings will comply with B&S “Vendors Instructions for Drawings”.  No pencil drawings will be accepted.  Drawings shall be CAD generated in full-scale geometry and scaled during printing, with scale in title block reflecting printing scale.

5.	  	B&S " Component Specification List " for the specific plant shall be followed.  Any deviation will require written approval by the B&S Project Leader.

6.	  	All valve solenoids shall be plug-in style.  Coils on solenoids shall have indicating lights, if available.

7.	  	Flow controls shall be meter-out type and be mounted in or near cylinder ports.  The use of manifold mounted flow controls shall be approved in writing by the B&S Project Leader.

8.	  	Equipment shall be provided with one main pneumatic connection point equipped with shut-off valve, filter, and regulator.  Shut-off valves shall purge air when turned to the closed or off position.  If necessary, a lubricator designed for the application shall also be provided following the regulator.

9.	  	Stackable, modular directional control valves shall be used.  The COMPANY shall provide 10-20% blank manifold stations on the equipment for future valve additions.  Blank stations shall be wired back to the control panel.  The wire shall be labeled as valve spare.

10.	  	All air blow-off lines shall be provided with a solenoid controlled shut-off valve so air is only used as needed.

11.	  	All valves shall have an identifying tag attached on or near the device to simplify trouble-shooting and maintenance.  All pressure controls and pressure switches shall have an identifying tag showing proper pressure setting.

PNEUMATIC SYSTEMS (Continued)

YES	NO

12. 	  	 Air purge lines for spindles or other mechanisms shall have a secondary 5 micron coalescing filter and regulator installed after the equipment's main filter/regulator but before the lubricator to insure dry oil-free air.

13. 	  	  All pneumatic systems shall be able to function on 75-psi compressed air.

X.   LUBRICATION SYSTEMS

1.	  	All equipment shall be furnished with a single point automatic lubrication system.  Separate systems shall be provided for each required lubricant type.  Oil mist lubrication requires the prior written approval of the B&S Project Leader.

2.	  	Reservoirs will have sufficient capacity to operate 120 hours, and be capable of filling without equipment shutdown.  Reservoirs shall have sight gauges for visual checking of oil level.

3.	  	Lubrication systems will have the following fault detection devices to automatically cycle-stop equipment and alert the operator of lube fault:

a.  Reservoirs shall have a low-level warning system.

b.  System shall have a high-pressure warning switch.

c.  Memory type pressure reset indicator pins shall be used on each lube outlet.

d.  Cycling switches shall be used on distribution blocks to monitor cycle.

4.	  	All tubing runs shall be supported.

5.	  	All reservoirs shall have a plate on the outside specifying the proper type oil to be used.  Both Mobil Oil and ISO specifications shall be included.

6.	  	Lubrication interval and frequency shall be controlled automatically by the equipment's PLC.  A manual lube push-button shall also be provided to test cycle the system and reset lube faults.

7.	  	Drawings and schematics together with a parts listing shall be part of the equipment documentation.  Parts list shall use the original manufacturers part numbers.  The Lubrication system drawings will comply with B&S “Vendors Instructions for Drawings”.  No pencil drawings will be accepted.  Drawings shall be CAD generated in full-scale geometry and scaled during printing, with scale in title block reflecting printing scale.

8.	  	The B&S "Component Specification List" for the B&S location where the equipment is being installed shall be followed.  Any deviation requires the prior written approval of the B&S Project Leader.

LUBRICATION SYSTEMS – Continued

YES	NO

9.	  	Reference “Lubrication List” in Appendix B for a list of common lubricants used in each B&S location.  Any lubricate, hydraulic oil, or grease shall conform to the list for normal applications.  As new lubricants are required, the written approval of the B&S Project Leader is required so “Lubrication List” can be revised.

XI.   MECHANICAL DESIGN

A.  BASIC EQUIPMENT

1.	  	Equipment shall be designed to be as small as possible.  Care shall be taken to make the load/unload side of equipment narrow to minimize equipment width.

2.	  	Manually loaded equipment shall have two-hand touch cycle start.  Additional devices (guarding, light screens, doors, etc.) for one-touch start shall be approved by B&S Safety Department.

3.	  	Equipment design shall allow access to Maintenance areas - motors, belts, ports, valves, pumps, electrical components, etc.  Visual inspection of level, pressure and other gages shall be possible without removal of guards.

4.	  	All guards shall be quick-release for rapid tool-change and maintenance access.  (Reference Safety Section)

5.	  	On machining systems slant all table and base surfaces to a common trough for good chip and coolant control.  There may be no pockets that can trap chips.   Mount fixtures, slides, spindles, etc. on small risers.  NO LARGE FLAT TABLES!

6.	  	A tool management system shall be integrated into equipment design that includes tool life counters for each tool or group of tools and broken or bad tool detection.  A signal light triggered by the tool counters shall be included to indicate if a tool is due for change.

7.	  	Equipment shall have andons (signal lights) visible from any side of the equipment to display equipment-operating status.  The B&S Project Leader shall provide specific requirements.

8.	  	Design shall permit so no damage occurs if equipment is clamped or cycled without a part.

9.	  	Mount spindles vertically when possible to improve coolant delivery to the bottom of the hole, and minimize equipment width.

MECHANICAL DESIGN

A.  BASIC EQUIPMENT  - Continued

YES	NO

10.	  	Proper motors shall be specified to run spindles continuous or start/stop as required.  Verify with Project Leader

11.	  	Spindle and slide accuracy / load capacities shall be verified and compatible with the application.  Maximize length to width ratio.  Tool overhang shall be within the spindle manufacturer’s recommendations where applicable.

12.	  	Dial or in-line transfer equipment shall provide adequate spacing between stations for ease of tool change and maintenance.

13.	  	All machining systems shall provide for containment and ducting of cutting fluids and chips.  All cutting fluid and chips shall return to a common point on the equipment.  The equipment shall be designed and plumbed to self-sufficiently clear chips in such a manner where they will not build up and cause potential malfunctions or quality defects.  The B&S Project Leader shall provide specific coolant system requirements.

14.	  	All fixture mounts, spindles, slides, etc. shall be positively located with dowels, keys or other methods to eliminate the possibility of misalignment or movement of components.

15.	  	Hoppers on automatic feeding systems shall be designed to have capacity of at least 4 hours based on requested production rates.

16.	  	Detail drawings of all special mechanical components shall be supplied with the equipment.  (See section IV, item # 1, "Documentation and Drawings" of these standards.)

17.	  	Provisions shall be made for lifting and handling machine, with critical lifting points marked, and all related equipment and tooling during installation.

B.  FIXTURE

1.	  	Fixtures shall be designed with automatic unload assist devices so load/unload can be accomplished using one hand.

2.	  	Part load orientation shall be determined by analyzing the best compromise between ease of loading, equipment width, and minimal re-orientation from preceding operation.

3.	  	Part location points shall be consistent for all operations.  3-2-1 datum scheme.

MECHANICAL DESIGN

B. FIXTURE  - Continued

YES	NO

4.	  	Rough locators shall be included to aid loading

5.	  	Clamping shall be directly over part locators to prevent bowing or distortion of parts.

6.	  	Devices shall be included to ensure parts are properly located and clamped.

7.	  	Fixtures shall be designed to eliminate need for blowing-off fixture – Prefer using gravity or coolant flush, slant fixture surfaces.  Design clearance for chips to fall through the fixture and eliminate chip-catching pockets.

8.	  	Use manifolds for hydraulic, pneumatic passages into fixture where feasible to minimize obstructive piping and plumbing cost.

9.	  	If fixture changeover is necessary equipment shall be designed for changeover times of less than two (2) minutes.  Changeover shall be done without the need for tools.  If screws are necessary, design so screws do not have to be removed completely.  Use same hex head/screw size as often as possible for all replaceable parts.  Use quick locators, doweled stops, fast adjustment, etc.  Include storage locations (out of the way) on equipment for the changeover parts.

10. 	  	Fixture locators shall be of the fixed renewable type and shall be replaceable without any need for grinding after assembly.  If some adjustment is necessary for initial equipment set-up, it   shall be of shim and block style using a single ground shim.

11.	  		All part fixtures shall have a “fool proof device” installed to prevent miss-loads.

12.	  	All parts and tool holding devices using air or hydraulic pressure shall be equipped with pressure sensitive switches to ensure the part or device is properly secured before the cycle can start.

XII.   TOOLING

YES	NO

1.	  	Commercially available standard tools shall be used whenever possible.  Indexable inserts shall be used when possible.  Contact the project leader for recommended suppliers that are common within B&S.

2.	  	Use quick change / easy adjust / preset tools.  Design for a maximum 2-minute tool change time.

3.	  	Where drill bushings or bushing plates are used, a slide tool change position or a bushing plate quick release mechanism shall be provided to assure quick tool change capability.

4.	 	A tool monitoring and management system shall be integrated into equipment design.  (See Section XI. item A.6.)

5.	  	Tool drawings are required on all non-standard tooling (see section V, "Documentation and Drawings") and includes standard tooling that is altered in any way.  A tool bill of materials list shall list the tool manufacturer’s name and part number.  Tool layouts do not qualify as tool drawings unless they contain all necessary information for making the actual tool(s).

XIII.  EQUIPMENT ACCEPTANCE

Two (2) successful equipment inspections and production run-offs shall occur. The first runoff will be at the COMPANY facility and the second runoff at the designated B&S facility.  Successful is defined as meeting or exceeding the COMPANY’S guaranteed production rates, cycle times, part quality, and equipment efficiency.

Any component, assembly, system or portion thereof failing to meet the requirements specified in, this document, shall be modified by the COMPANY, at COMPANY’s expense, before proceeding with equipment acceptance testing.

The (2) run-offs shall be structured in the following sequence.

YES	 NO

1.	  	    B&S will furnish all raw materials and/or parts for the production run-off.  B&S will warrant the quality of the material to the extent that if the material is not to specification, it will be replaced as rapidly as possible to facilitate the run-off.  Once the material is in the COMPANY’s plant, the COMPANY is responsible for assuring the material remains in an acceptable condition.  All parts run during equipment testing and tryouts are to be accounted for by the COMPANY with documented inspection results.

EQUIPMENT ACCEPTANCE (Continued)

2. 	  		   The COMPANY shall review equipment against all acceptance criteria and notify B&S when it is confident the equipment is ready for inspection and run-off.  All pallets/fixtures shall be dimensionally verified and corrected prior to any machine capability studies.  At this time the COMPANY shall supply a minimum of ten (10) sample parts, or a minimum of three (3) parts/fixture for multi-fixture equipment, to B&S Project Leader for evaluation by B&S Quality Assurance personnel.

The evaluation criterion is as follows: The Ppk shall meet a 1.67 for Critical (C) blueprint requirements.  A Ppk of 1.33 is mandatory for all Major (M) requirements, and other pre-determined critical product features.  A 1.0 Ppk is required for all other features that are measured as a variable.  All Attribute requirements shall be within the blueprint tolerances, including visual attributes (e.g. burrs, chatter, clamping damage, etc.)  Upon verification of the sample’s part quality, B&S will send personnel to the COMPANY’s facility for inspection and run-off.  NOTE: Ppk calculations use sigma based on individuals (RMS).  Sigma based on Rbar/d2 is not to be used to calculate process capability.

3.	  		An equipment inspection will be conducted by B&S personnel, at both the COMPANY’s facility and at the B&S plant, for complete adherence to this document and the “Component Specification List” as agreed upon at the time of the equipment order.

4.	  		The B&S Project Leader, Quality Assurance, Safety, and Industrial Controls departments will check the equipment’s functionality at both the COMPANY’s facility and the B&S facility.

These checks will verify the performance of all control & diagnostic functions including:

a.	Equipment safety guards and devices

b.	Equipment probes, tool detection, and in-process gages

c.	Hydraulic system, pneumatic system, automatic lubrication system

e.	Electrical control system

f.	Equipment cycle time

- Mechanical components

XIII.  EQUIPMENT ACCEPTANCE – (continued)

YES	 NO

5.	  	Prior to both run-offs, a pre-qualification test will be conducted before a run-off can take place.  To complete the pre-qualification test, the equipment shall pass the following criteria:

a.	Dry Run:

The equipment is to operate for a minimum of two (2) hours without parts.  During this period absolutely no equipment faults, stoppages, or breakdowns are allowed, for any reason.  Equipment shall have 100% uptime.

b.	Q.C. Check 1:

Following the successful Dry Run, between three (3) and ten (10) parts per fixture will be processed in the equipment and inspected by B&S personnel.  Measured dimensions of each part shall maintain the appropriate Ppk requirement.  Make adjustments as necessary.

c.	Conditioning Run:

Equipment will be run one (1) hour processing parts with absolutely no downtime allowed for any reason.  Equipment shall have 100% uptime and produce at the required cycle rate.  With the exception of manual load/unload, if required, the equipment shall run without any outside intervention.  Equipment shall function properly and demonstrate its ability to self-sufficiently control any machining chips generated.

d.	Q.C. Check 2:

Upon a successful Conditioning Run, between three (3) and ten (10) parts per fixture will be processed in the equipment and will be inspected by B&S personnel.  Measured dimensions of each part shall maintain the appropriate Ppk requirement.  Make adjustments as necessary. The difference between Q.C. check 1 and Q.C. check 2 shall be noted.

e.	Equipment Shut Down Period:

The equipment will be shut down for a minimum of 30 minutes to study the effects of thermal growth and equipment shutdown/restart on part quality.

f.	Equipment Restart:

Equipment will be restarted and the first three (3) parts will be processed complete without any adjustment to the equipment.

EQUIPMENT ACCEPTANCE – (continued)

g.	Q.C. Check 3:

B&S personnel will inspect the parts processed during the Equipment Restart.  Measured dimensions of each part shall maintain the appropriate Ppk requirement.  Make adjustments as necessary. The differences between

Q.C. Check 1, Q.C. check 2, and Q.C. check 3 shall be noted and shall demonstrate a consistent pattern.

Upon successful completion of the pre-test, the run-off may begin.

XIII.  EQUIPMENT ACCEPTANCE  - (continued)

YES	NO

6.	  		Run-off, at both the COMPANY’s facility and the B&S plant is done to determine the equipment’s capability and to determine statistical control.  Run-off will proceed as follows:

  - Equipment will run, producing parts, for two (2) hours.  Equipment shall demonstrate 100% uptime.  Adjustments and downtime is limited to scheduled tool change only.

  - Unless provided by B&S, gages or measuring instruments shall be of the variable type with graduations not coarser than 10% of the tolerance.

  - A log, shall be kept by B&S personnel, of the events that could influence the equipment capability, such as tool changes, measurement problems, raw material discrepancies, coolant flow variations, chip removal problems, air line pressure fluctuations, and so forth.

  - In the case of multi-pallet or fixture machines such as transfer lines or dials;

      - A minimum of (3) parts per fixture/pallet should be made by the COMPANY and the study shall be made available to B&S personnel.

      - All pallets/fixtures will be equally represented during the run-off.

e.	The sampling plan will be relative to the cycle time.  Unless otherwise agreed upon, the sampling plan will be based on:

      - The total sample size for evaluation of an operation/station will be a 50-piece minimum capability study when a 1 or 2 fixture machine is studied.

      - Sample size for qualifying multiple fixtures/pallets will be a minimum of three (3) to ten (10) parts per fixture/pallet for selected features to assure all lock up positions will be adequately studied as agreed upon by both parties.

The sample size will be selected from a population of parts produced during a minimum of a two (2) hour run-off.  The sampling plan will define the amount of (filler) parts to be produced between each subgroup/round.

EQUIPMENT ACCEPTANCE  - (continued)

f.   B&S will measure the agreed upon features, document each individual reading and summarize the results of each feature into in a histogram including the following categories:

Average – statistical mean of feature.

Offset from the Nominal – a distance the mean (X-bar) moves away from nominal (expressed as a percentage of the tolerance).

Range – difference between highest and lowest reading.

Statistical Control Analysis – statistical control shall be maintained in order to accurately evaluate capability.  Statistical control can be evaluated by using the moving range from a run chart or the range from an X Bar-R chart.  If an out of control situation exists, the assignable causes need to be identified and eliminated.  In addition, a method should be established to prevent reoccurrence.  Further data collection may be required.

6 sigma – plus or minus 3 standard deviations.

Ppk – is the number of ½ bell shaped curves actually fit between the process average and the closest specification limit.  Ppk is an index of process performance that tells how well a system is meeting specifications.  Ppk calculations use sigma based on individuals (RMS) and shows how the system is actually running when compared to the specifications.  This index also takes into account how well the process is centered within the specification limits and compares the result to the closest side of the specification limit.

Pp – is the number of full bell shaped curves that could fit between the upper and lower specifications  (Pp does not indicate that parts are within specification).  Pp is an index of process performance that tells how well a system could meet two sided specifications.  Like Ppk it uses sigma calculations based on individuals (RMS).

g.	B&S will evaluate the Capability Study using the following criteria for measurable features:

      - 1.67 Ppk for all “Critical” (C) blueprint characteristics.

      - 1.33 Ppk for all Major (M) requirements, and other pre-determined critical product features.

      - 1.0 Ppk for all other blueprint characteristics.

- NOTE: Ppk calculations use sigma based on individuals (RMS).  Sigma based on Rbar/d2 is not to be used to calculate process capability.

- When Ppk’s do not meet the above criteria due to other manufacturing requirements that dictate a feature (or tool) shall be biased toward one side of the specification, a decision may be made to use alternate indices.  The details and reason for this

decision shall be stated with written approval from B&S manufacturing and quality representatives.

EQUIPMENT ACCEPTANCE  - (continued)

If the equipment fails to comply with any item on the pre-test, COMPANY run-off, and/or B&S run-off, the COMPANY, at COMPANY’s expense, shall correct the problem.  Also, the problem shall be corrected in a manner that prevents the problem from happening again and in a manner satisfactory to the B&S Project Leader.  Then the equipment acceptance will restart at the appropriate step, as determined by the B&S Project Leader.

XIV. STANDARDS DEVIATIONS REQUEST

As detailed in the QUOTATION INSTRUCTIONS section provide reasons for each “no” answer below.

1.

2.

3.

4.

5.

6.

7.

8.

9.

10.

XV.  COMPANY PROFILE SHEET

IMPORTANT: The Company shall complete this page.

Suppliers Legal Name:

State of Incorporation:

Address:

Office Phone No.

Email Address:

CEO/President’s name:

CEO/Presidents direct phone number at office:

CEO/Presidents direct phone number at home:

Ownership of Company:

FOR EACH OF THE FOLLOWING AREAS, PLEASE PROVIDE:

Name		Phone  	Email		Pager          Cell Phone        Fax

Inside Sales:

Outside Sales:

Service Dept:

(Parts)

Tech. Support:

Training Dept:

Other:

PERSON MAKING COMMITMENT ON BEHALF OF THE SUPPLIER:

Print Name:

Title:

Date:

Signature:

XV.  COMPANY PROFILE SHEET (Continued)

THE COMPANY WILL COMMIT THE FOLLOWING PERSONNEL TO PROVIDE (24) HOUR, (7) DAYS A WEEK TECHNICAL SUPPORT AND ACCESS TO NECESSARY PARTS TO SUPPORT OPERATIONS FOR B&S.

PRIMARY CONTACT:

NAME:

WORK PHONE:

PAGER:

CELL PHONE:

HOME PHONE:

SECONDARY CONTACT:

NAME:

WORK PHONE:

PAGER:

CELL PHONE:

HOME PHONE:

Appendix “A”

Component Specifications

|  | COMPANY WIDE STANDARD |  | PROJECT LEADER DEVIATION |  |
| --- | --- | --- | --- | --- |
|  | MFG | MODEL | MFG | MODEL |
| ELECTRICAL |  |  |  |  |
| Broken Tool Detectors | Leukhardt | BK Mikro 4 Special Applications |  |  |
| Tool Wear Monitors | Techna Tool | Techna Check |  |  |
| Cable Pull Switches | Rees |  |  |  |
| Cam Switches | Allen-Bradley | Bulletin 803 |  |  |
|  | Press Applications | Gemco |  |  |
| Circuit Breakers | Square D |  |  |  |
| Control Buttons | Allen-Bradley | Bulletin 800T |  |  |
| CNC Controls | Approval Required |  |  |  |
| Cycle Start Buttons | Banner | Model #OTBA5 |  |  |
| DC Power Supplies | Sola | SDN-4-24-100P |  |  |
|  |  | Energy Limiting |  |  |
| Disconnects | Allen-Bradley | Bul 1494 V/F Flange Mount Only |  |  |
| Electric Heaters | Chromalox |  |  |  |
|  | Fastheat |  |  |  |
| Enclosures | Hoffman | NEMA Type 12, 4X |  |  |
| Encoders | Dynapar |  |  |  |
| Flow Switch | Efector | SI Series |  |  |
| Fuses | Buss | (LPJ Low Peak Required) |  |  |
| Fuse Holders (Motors) | Buss | JTN |  |  |
| Light Curtains | Banner | Mini-Screen |  |  |
| Light Stacks | Allen-Bradley | Bul 855T |  |  |
| Limit Switches | Micro Switch | 914 Series |  |  |
|  | Allen-Bradley | Bulletin 802T, 802M |  |  |
| Man-Machine Interface | See Project Leader |  |  |  |
| Motors (TEFCEE) (Use energy-efficient wherever possible) All motors to have sealed bearings. | Baldor |  |  |  |
|  | Demag |  |  |  |
| Motor Starters | Allen-Bradley | Bulletin 509, 505 |  |  |
| Multiple Limit Switches | Euchner |  |  |  |
| Overload Relays | Allen-Bradley | Bulletin 193 |  |  |
| Photo Electric | Banner | Multi-Beam |  |  |
|  | Allen-Bradley |  |  |  |
|  | Efector | Diffuse Style Switches |  |  |
| Pilot Lights | Allen-Bradley | Bulletin 800T (push-to-test) |  |  |
| Panel Air Conditioning (R-134A) | McLean Midwest |  |  |  |
| Power Conditioners | Sola |  |  |  |
| Pressure Switches | Efector | PN Series |  |  |
| Programmable Controllers | Allen-Bradley | SLC 500 Modular |  |  |
| Proximity Switches | See Project Leader |  |  |  |
| Relays | Allen-Bradley | Bulletin 700P |  |  |
| Servo Drives & Motors | See Project Leader |  |  |  |
| Spindle Drives | Approval Required |  |  |  |
| Temperature Controllers | Honeywell |  |  |  |
| Terminal Blocks | Allen-Bradley | Bulletin 1492 |  |  |
| Terminal Block (Fused) | Allen-Bradley | Bulletin 1492 |  |  |
| Timers-Electronic | Square D | Solid State |  |  |
|  | Eagle Signal | HP5 Cycle-Flex Series |  |  |
|  |  | HP7 Repeat Cycle |  |  |
|  | Potter Brumfield | Depends on Application |  |  |
| Transformers (dry type) | Square D | Enclosed/External Mount |  |  |
|  | Heavy Duty | Enclosed/External Mount |  |  |
| Vacuum Switches | Square D |  |  |  |
| Variable Frequency Drives | See Project Leader |  |  |  |
| Vibratory Bowl Control | Rikart |  |  |  |
|  | Rodix |  |  |  |
|  | Staco |  |  |  |
| HYDRAULIC |  |  |  |  |
| Cylinders | Parker Hannifin | 2H Series |  |  |
| Swing Clamps | See Project Leader |  |  |  |
| Work Supports | See Project Leader |  |  |  |
| Air/Oil Boosters | See Project Leader |  |  |  |
| Pumps | Vickers |  |  |  |
| **High Pressure Pumps |  |  |  |  |
| Directional Valves | Vickers |  |  |  |
| Misc. Components | Vickers |  |  |  |
|  | Parker |  |  |  |
| Specialty Components | Parker |  |  |  |
|  | Sun |  |  |  |
|  | Gresen |  |  |  |
|  | Fluid Controls |  |  |  |
| Filters | Hycon |  |  |  |
|  | Vickers |  |  |  |
| Manifolds/Subplates | PHD |  |  |  |
|  | Daman |  |  |  |
|  | Sun |  |  |  |
| Flareless Fittings | See Project Leader |  |  |  |
| Hyd. Hose Connections | Parker |  |  |  |
| Systems | Trabon | Lubriquip MS Series |  |  |
| Cylinders | Parker-Hannifin (preferred) |  |  |  |
|  | Bimba |  |  |  |
| Filter Regulators | Norgren Modular |  |  |  |
| Air-Line Lubricators | Norgren Modular Mist |  |  |  |
| Directional Valves | Numatics | Mark 8 or 15, Manifold Mount |  |  |
|  | Asco |  |  |  |
| PLUMBINBG/WATER SYSTEM |  |  |  |  |
| Heat Exchangers | B&G | Shell & Tube |  |  |
|  | American Standard | Shell & Tube |  |  |
| Pumps | Ingersoll-Rand |  |  |  |
|  | Usher | Series 5562 |  |  |
|  | B&G |  |  |  |
| Ball Valves | Apollo |  |  |  |
|  | Parker-Hannifin |  |  |  |
| Butterfly Valves | Keystone |  |  |  |
|  | Stockham |  |  |  |
| High Pressure Coolant Valves | See Project Leader |  |  |  |
| Low Pressure Coolant Valves | ASCO |  |  |  |
| Manual Valves | Crane | Gate, Globe Plug |  |  |
| Rated 125# WOG | See Project Leader |  |  |  |
| Basket Strainers | Mueller |  |  |  |
|  | Hayward |  |  |  |
| STEAM SPECIALTIES |  |  |  |  |
| Traps, Strainers, Etc. | See Project Leader |  |  |  |
| Valves | See Project Leader |  |  |  |
| Temperature Gauges | Taylor |  |  |  |
|  | H.O. Trerice |  |  |  |
| NATURAL GAS EQUIPMENT |  |  |  |  |
|  | Maxitrol |  |  |  |
| PLASTIC PIPE & PVC |  |  |  |  |
|  | Hayward |  |  |  |
|  | Plastomatic |  |  |  |
| COOLANT ENVIRONMENT |  |  |  |  |
|  | Viton | Seals |  |  |
|  | Lexan | Plastic Sheet |  |  |
| MECHANICAL |  |  |  |  |
| Spindles | See Project Leader |  |  |  |
| (Air purge required in coolant environments) |  |  |  |  |
| Quill Units | See Project Leader |  |  |  |
| Drill Heads |  |  |  |  |
| Slides | Setco |  |  |  |
|  | Gilman |  |  |  |
| Gas Shocks | Dadco |  |  |  |
| Gear Reducers | Browning |  |  |  |
| Tool Holders | Scully-Jones |  |  |  |
|  | T.M. Smith |  |  |  |
| (Whenever possible use standard tooling available from multiple sources) |  |  |  |  |
| PAINT |  |  |  |  |
|  | Pittsburgh Paints |  |  |  |
| Primer | Multi-Prime Primer |  |  |  |
|  | No. 97-682 Color Red |  |  |  |
|  | No. 97-684 Color White |  |  |  |
|  | Use with aquapon epoxy, fast dry enamels, brush or roller enamels and pitthane urethane enamels. |  |  |  |
|  | EPOXY PAINT (Preferred Material) |  |  |  |
|  | Aquapon-Polyamide-Epoxy Coating |  |  |  |
| Machines | See Project Leader |  |  |  |
| Guards | Safety Yellow |  |  |  |
|  | No. 97-13 with component ‘B’ No. 97-98 epoxy |  |  |  |
| Electric Panel | External – same as machine internal white |  |  |  |
| BRUSH, ROLLER, OR SPRAY ENAMELS | Non-hostile environments only. Equipment exposed to cutting fluids, solvents, washer chemicals, etc., shall be painted w/epoxy paints specified above) |  |  |  |
| Machines | See Project Leader |  |  |  |
| Guards | Safety Yellow No. 54-308 |  |  |  |
| Electric Panel | Same as Machine |  |  |  |
| FAST DRY SPRAY ENAMEL | (Non-hostile environments only. Equipment exposed to cutting fluids, solvents, washer chemicals, etc., shall be painted w/epoxy paints as specified above) |  |  |  |
| Machines | See Project Leader |  |  |  |
| Guards | Safety Yellow No. 97-13 |  |  |  |
| Electric Panel | Same as machine |  |  |  |

APPENDIX “B”

LUBRICATION LIST

| PRODUCT OILS | ISO GRADE | B&S PART # | MANUFACTURER |
| --- | --- | --- | --- |
| Lubricant HF-28HYDR-Fluid |  | 1200163120 | Chem-Trend Inc. |
| 133K Anti-Seize lubricant |  | 1200167020 | Loctite Corp. |
| 603936-00 Mobil DTE Excel 68 |  | 2000024572 | Exxonmobil Corp. |
| A-371-A Stamping Compound |  | 1200142440 | Advance Technique Co |
| Acculube LB-2000 type 1 gal Size |  | 1601500670 | Unknown Mfg. (only) |
| Alvania EP-2 Shell |  | 1760066862 | Shell Oil Products |
| AP-1 Hi-Temp Non-Melting Grease |  | 2000011346 | Jet-Lube Inc. |
| C5-A Copper base anti-seize paste 51006 |  | 2000026462 | Henkel Loctite Corp. |
| CDL-2848 Hi Temp Synthetic Lubricant |  |  |  |
| Chevron Way Oil Vistac D ISO 220 |  | 2000025362 | Chevron Products Co. |
| Compound 765-1151 Anti-Seize |  | 1760019990 | Napa Auto |
| Die Slick No 7 Anti-Solder |  | 2000018366 | G.W. Smith & sons Inc. |
| Dow Corning #111 Compound HV5.3 oz. Tube |  | 1601531010 | Dow Corning Corp. |
| Drive Clean HD 30 |  | 1200131510 | Exxonmobil Corp. |
| Extrudoil 6636 |  | 2000023397 | Fuchs Lubricants Co |
| Kluber Lube 206546 50 gram tube |  | 1760018210 | Kluber |
| Kluber Lube 206878 1 kilo can 1000 G |  | 1760018200 | Kluber |
| Kluber Lube NBU-15 ISOFLEX 400 gram cart |  | 1760018220 | Kluber |
| Loctite anti-seize 76759 |  | 1601503930 | Loctite Corp |
| Loctite Pneumatic/hydraulic 545 thread S |  | 1800007520 | Loc-Tite |
| LPS1 Greaseless Lube |  | 1200135000 | LPS Laboratories |
| Lube #3 extreme pressure Lube #3 |  | 1601511190 | Chicago Mfg. & Dist. |
| Lube Bori-Lube 10 polishing lubricant |  | 1601511220 | Not available |
| Lube chain & cable JAX #104 |  | 1601511700 | Jax Lubricant |
| Lube chain & light oil part #00202 |  | 1620203740 | Sprayon |
| Lube-line HT-1300 |  | 2000004468 | Lube-Line Corp |
| Lube-line LT-450 |  | 2000004469 | Lube-Line Corp |
| Lubricant *M1 all purpose 5 gallon |  | 1760127880 | Starrett L S Company |
| Lubricant *M1 all purpose 12 oz. |  | 1200165710 | Starrett L S Company |
| Lubricant *Mobilgear 632 |  | 1200131270 | Exxonmobil Corp. |
| Lubricant *Mobilgear 634 |  | 1200131280 | Exxonmobil Corp |
| Lubricant * Mobilux EP 1 |  | 1200131460 | Exxonmobil Corp |
| Lubricant * Velocite oil #10 | 22 | 1200131660 | Exxonmobil Corp. |
| Lubricant * Velocite Oil #6 | 10 | 1200131650 | Exxonmobil Corp. |
| Lubricant 8 ox liquid wrench L1-08 |  | 1760000610 | Murray Supply |
| Lubricant Acculube Jell TB-8 |  | 1200165610 | Accu-Lube |
| Lubricant Chain BK 765-1370 |  | 1760012122 | Napa Auto |
| Lubricant Cosmolubric HF-130 |  | 1200120190 | E.F. Houghton & Co. |
| Lubricant CRC 3-36 Multi Purpose |  | 1601519700 | CRC Chemicals |
| Lubricant LPS #1 Greaseless 12 Gallon |  | 1601570620 | LPS Laboratories |
| Lubricant Lubriplate No. 110 34 Lb. Pail |  | 1601599190 | Unknown Mfg. (only) |
| Lubricant Mobil gear 626 | 68 | 1200132130 | Exxonmobil Corp. |
| Lubricant Mobilmet Omicron |  | 1200131380 | Exxonmobil Corp. |
| Lubricant MP-121 Parco |  | 1810103210 | Not Available |
| Lubricant Oil 3-in-one |  | 1130159510 | WD-40 Company |
| Lubricant Oil Vactra No. 2 |  | 1200131600 | Exxonmobil Corp. |
| Lubricant Penetrant Plus Demoisturant 41 |  | 1130161210 | Percy Harms Corp. |
| Lubricant Power Lube PLP-232 |  | 2000008387 | Chem-Trend Inc. |
| Lubricant Rubber Emulsion P-80 CAT #5393 |  | 1200160770 | Int. Products Corp. |
| Lubricant Rubber Emulsion P-80 CAT #5391 |  |  |  |
| Lubricant Spectasol Q110 |  | 1200180640 | American Chemical |
| Lubricant TKX #02016: 11 oz. |  | 1601580300 | LPS Laboratories |
| Lubricant VIS96-C |  | 1200180100 | Colburn |
| Lubricant WD-40 55 gallon size |  | 120016136H | Unknown mfg. (only) |
| Lubricant WD-40 all purpose 9 oz. |  | 1200161360 | WD-40 |
| Lubricant white oil 22 |  | 1200166790 | Texaco |
| Lubricant wire pull fast draw 1 qt. |  | 1641721620 | Greenlee |
| Lubricant wire pulling, part # GARB |  | 1641731250 | Butler Supply |
| Lubricoat Boron Nitride |  | 1601511570 | Pyronics Corp. |
| Lubriplate No. 3000W |  | 2000019445 | Fiske Brothers |
| Misty Ultra High Temp Syn Blend (14oz) |  | 1200165740 | Amrep Inc. |
| Mobil DTE Excel 46 (55 gallon drum) |  | 1200167100 | Exxonmobil Corp. |
| Mobil Vactra Oil No. 2 |  | 2000024351 | Exxonmobil Corp. |
| Mobil Vactra Oil No. 4 |  | 2000024352 | Exxonmobil Corp. |
| Mobilfluid 424 |  | 1200164100 | Exxonmobil Corp. |
| Mobilgear SHC 3200 |  | 1200164390 | Exxonmobil Corp. |
| Oil 707 special |  | 1200165730 | E.F. Houghton & Co. |
| Oil Air Tool |  | 1601511620 | McMaster-Carr |
| Oil Assembly Oil Mobil |  | 1200162410 | Exxonmobil corp. |
| Oil ATF Multi-Purpose |  | 1200131010 | Exxonmobil Corp. |
| Oil Benzgrind HP22 grinding |  | 1200180180 | Benz Oil |
| Oil Chain Lubricant 601 Chesterton 12 oz. |  | 1601511410 | Chesterton |
| Oil Cutting 78K80102 Chesterton 14.5 oz. |  | 1601511800 | A.W.Chesterton Co. |
| Oil Cutting Thread Ridgid Dark EDP#7083 |  | 1601510380 | Thew, W.E. Supply Co. |
| Oil Dark Threading 1 gallon |  | 1760022642 |  |
| Oil Delvac 1200 super 10W30 |  | 1200164110 | Exxonmobil Corp. |
| Oil Delvac 1330 Mobil |  | 1200163350 | Exxonmobil Corp. |
| Oil Die slick #RDL 9720 |  | 1810203780 | Chem-Trend Inc. |
| Oil DTE 24 Mobil | 32 | 1200131100 | Exxonmobil corp. |
| Oil DTE 25 Mobil | 46 | 1200131110 | Exxonmobil Corp. |
| Oil DTE 26 Mobil | 68 | 1200131120 | Exxonmobil Corp. |
| Oil DTE 832 |  | 1200167140 | Exxonmobil Corp. |
| Oil DTE BB Mobil | 220 | 1200164200 | Exxonmobil Corp. |
| Oil DTE Heavy Medium | 46 | 1200163130 | Exxonmobil Corp. |
| Oil DTE Heavy Mobil | 150 | 1200131080 |  |
| Oil DTE HH |  | 1200165700 | Exxonmobil Corp. |
| Oil DTE Light Mobil | 46 | 1200131090 | Exxonmobil Corp. |
| Oil Fomblin Y-L-VAC-25/6 4Lb. |  | 1131130590 | Exxonmobil Corp. |
| Oil Halovac 190 |  | 1200163480 | Exxonmobil Corp. |
| Oil Hydraulic Oil 150 mobil AW46 |  | 1200131830 | Dupont |
| Oil Lubricant F442 |  | 1601500060 | Vacuume Fluid Inc. |
| Oil Lubricast 852 |  | 1810203590 | Exxonmobil Corp. PPB DB-DUP S/I-DEL CHEMTOOL |
| Oil Mobil Velocite #3 | 2 | 1200181080 | Exxonmobil Corp. |
| Oil Mobilmet Gamma |  | 1200131390 | Exxonmobil Corp. |
| Oil Mobilmet NU |  | 1200180300 | Exxonmobil Corp. |
| Oil Mobilmet S-122 |  | 1200165410 | Exxonmobil Corp. |
| Oil Mobilube HD 80W/90 |  | 1200131450 | Exxonmobil Corp. |
| Oil Parquench 90XN |  | 1200163400 | Park Chemical Co. |
| Oil Petro-Hone 95 |  | 1200165330 | Petron Corporation |
| Oil Quench Thermisol K65 |  | 1200162970 | Fuchs Lubricants |
| Oil RDL-5002 Start-Up |  | 2000010326 | Chem-Trend Inc. |
| Oil SHC 630 Mobil |  | 1200164850 | Exxonmobil Corp. |
| Oil SHC-634 Mobil |  | 1200163050 | Exxonmobil Corp. |
| Oil SHC-624 Mobil |  | 1200131970 | Exxonmobil Corp. |
| Oil Vactra 2 |  | 1200165450 | Exxonmobil Corp. |
| Oil Vactra 2 Mobil |  | 1200164280 | Exxonmobil Corp. |
| Oil Vactra BB |  | 1200131590 | Exxonmobil Corp. |
| Oil Vactra Extra Heavy |  | 1200131630 | Exxonmobil Corp. |
| Oil Vactra Medium Mobil |  | 1200162820 | Exxonmobil Corp. |
| Oil-Air Tool *F 442 32 oz. |  | 1200165810 | Schrader Bellows |
| Oil-Tap Magic Aluminum Thread Cutting |  | 1200166330 | Steco |
| Parco Lubricant PL-101 260071 |  |  | Henkel Surface Tech. |
| Paste Molykote G-N 2.8 oz. Tubes was “U” |  |  | Dow corning |
| Paste-Teflon 023030 4 oz. |  |  | Harvey’s |
| Perchem Draw Die compound 3520-BS-1 |  |  | Perkins Products |
| Ridgid Dark Thread Cutting Oil 70830 |  |  | Rigid-Ridge Tool Co. |
| Silicone Clear RTV Adhesive Sealant SI |  |  | Grainger, W.W. Indus |
| Silicone Gasket #2 Non-Hardening 11 oz. |  |  | Grainger, W.W. Indus |
| Silver Grade anti-Seize Lub (.4 oz. Can) |  |  | Henkel Loctite Corp. |
| Silver Grade Anti-Seize Lub (1 lb.) 76764 |  |  | Loctite Corp. |
| Super Lube (‘R) Synthetic-Based W/PTFE |  |  |  |
| Syntilo 9930 |  |  | Castrol |
| Tap Magic Formula 1 8358 |  |  | Setco corp. |
| Tap Magic Protap Fluid cutting (16 oz.) |  |  | Setco |
| Tellus 32 |  |  | Shell Oil Products |
| Tellus Oil T 32 65401 |  |  |  |
| Tri-Flow Industrial Lubricant W/Teflon |  |  |  |
| Valcool VNT 900 |  |  |  |
| Wax Bees #1100K91 M/Carr |  |  | Apex |
| WD-40 1 gallon container |  |  | WD-40 Company |
| WD-40 1 gallon container |  | 1760001060 | WD-40 Company |
| WD-40 Bulk Liquid |  | 1200166390 | WD-40 Company |
| WD-40 Bulk Liquid WY3-155B |  | 2000022109 | Castrol Industrial |
| PRODUCT – GREASE |  |  |  |
| Grease #2 Zeniplex 14 oz. |  | 1760001640 | X-Unknown Mfg. |
| Grease * L0044-092 (Libriplate 1) |  | 1760049940 | Lubriplate |
| Grease Bearing Hi-Temp. NHTC-14 (Bostik) |  | 1601512570 | Bostik Findley Inc. |
| Grease Cartridge 12.5 oz. Mobil SHC 220 |  | 1200131300 | Exxonmobil Corp. |
| Grease Cartridge 14.0 oz. Darina Z |  | 1200125010 | Exxonmobil Corp. |
| Grease Cartridge 14 oz. Mobilith SHC 100 |  | 1301614470 | Grainger, W.W. Indus |
| Grease Cartridge 3 oz. Multi-Purpose MCRR# |  | 1301614140 | Ultra Lube Products Co. |
| Grease Chevron SRI #2 |  | 1301614460 | Unknown Mfg. (only) |
| Grease Dielectric Silicone 3 oz. Tube |  | 120162270 | McMaster-Carr Supply |
| Grease EDM 244 B/S |  | 1200162940 | Electrodes Inc. |
| Grease ISOFLEX #NBU-15; 50 gram tube |  | 1130630020 | Kluber Lubrication |
| Grease Isoflex #NBU-15; Qt Cans #004026 |  | 1130630180 | Kluber Lubrication |
| Grease Isoflex Topas NB l52 |  | 1131801170 | Kluber Lubrication |
| Grease Isoflex Topas NB 52 |  | 1130639880 | Kluber Lubrication |
| Grease Lithium Multi-Purposey (l lb.) |  | 1601512290 | McMaster-Carr Supply |
| Grease Lubecon I/M 200 (20 Liter Bucket) |  | 1200166560 | Lubecon |
| Grease Mobilgear 629 |  | 1200131260 | Exxonmobil Corp. |
| Grease Mobilgear 630 Mobil |  | 1200131740 | Exxonmobil Corp. |
| Grease Mobilgear 636 |  | 1200165850 | Exxonmobil Corp. |
| Grease Mobilgrease XHP222 (14 oz. Tube) |  | 1200163020 | Exxonmobil Corp. |
| Grease Mobilith AW 1 |  | 1200162590 | Exxonmobil Corp. |
| Grease Mobilith SHC 100 |  | 1200165860 | Exxonmobil Corp. |
| Grease Mobilith SHC 460 |  | 1200132110 | Exxonmobil Corp. |
| Grease Mobiltemp SHC 32 |  | 1200131430 | Exxonmobil Corp. |
| Grease Multi Purpose Lithum |  | 1760005830 | Ace Hardware Murray |
| Grease-Isoflex NBU-15 (50 grams) |  | 1200161470 | Kluber Lubrication |
| Grease-Metal GN 50 lb. |  |  |  |
| Grease-Metal GN 50 lb. |  | 1200164980 | DOW |
| Mobilgrease Special XHP222 |  | 1200131290 | Exxonmobil Corp. |
| Mobilgrease XHP 222 special |  | 2000025787 | Exxonmobil Corp. |
| Festo-Gel super Lube Grease |  | 2000004473 | Synco Chemical Corp. |
| Mobilith AW-2 |  | 1200162660 | Exxonmobil Corp. |
| PRODUCT - MOTOR OILS |  |  |  |
| 481168-00 Mobil 10W-30 98868P |  | 2000024559 | Exxonmobil Corp. |
| Exxon XD-3 10W30 |  | 2000021372 | Exxonmobil Corp. |
| Exxon XD-3 20 Oil |  | 2000019357 | Exxonmobil Corp. |
| Exxon XD-3 30 |  | 2000020674 | Exxonmobil Corp. |
| Oil Motor 10W-30 Mobil 1 Tri-Synthetic |  | 1200106210 | Exxonmobil Corp. |
| Oil Motor 10W-40 Havoline Formula3 26420 |  | 1200166360 | Chevron Texaco Corp. |
| Oil Motor 10W-40 |  | 1650018372 | Oil |
| Mobil 1 15W50 |  | 2000028900 | Exxonmobil Corp. |
| Oil 30SAE Shell X-100 |  | 1760009890 | Shell Oil Products |
| Oil Motor 30W Heavy Duty 75-110 |  | 1760000550 | Napa Auto |
| PRODUCT - LUBRICANT SPRAYS |  |  |  |
| C5-A Copper Base Anti-Seize Aerosol 16oz |  | 2000026416 | Henkel Loctite Corp. |
| Lubricant Acculube Spray PS-8 8 oz. Spray |  | 1200165620 | Accu-Lube |
| Lubricant Spray LPS 1 Greaseless 11 oz. |  | 1601511290 | See Thew (Shad&boyd) |
| Lubricant WD-40 15 oz. Spray |  | 1601511320 | D-40 Company |
| Graphite Spray 10 oz. |  | 1760005570 |  |
| Oil Penetrating Kano Kroil 10 oz. Spray |  | 1601511790 | Kano Laboratories |
| Lubricant Spray LPS 2 General Purpose 73 |  | 1601511780 | LPS Laboratories |
| Lubricant Spray LPS 3 HV Weight |  | 1601511300 | See Thew (Shad&Boyd) |
| Lubricant Spray Starrett #M1 #93215 12 0 |  | 1601511390 | PPB DB-DUP S/I-DEL |
| Oil Drive Clean 5W30 |  | 1200164580 | Exxonmobil Corp. |
| Oil Mobil 1 5W30 |  | 1200132070 | Exxonmobil Corp. |
| Spray Lubricant Magna-Plate Lubricant JA |  | 1601504660 | Jax Lubricant |
| Spray Molykote 1254K31 G-N Spray Lubricant |  | 1601513280 | Dow Chemical Corp. |
| Lubricant/Penetrant 20007 Tri-Flow 12 oz. |  | 1601511360 | See Thew (Shad&Boyd) |
| WD-40 12 oz. Spray on Lube |  | 1760000010 | WD-40 Company |
| PRODUCT – COOLANT / CUTTING FLUIDS |  |  |  |
| Coolant Syntilo 9913 Castrol Synt |  | 1200180860 | Castrol Industrial |
| Dura-Kool M |  | 2000008041 | Benz Oil |
| Ecocut AB |  | 2000027490 | Fuchs Lubricants Co. |
| Fluid Cool ToolII 16 oz. LK50 |  | 1601511130 | Tools & Abrasives |
| Hocut 795 |  | 2000002170 | Houghton Internation Inc. |
| Tech Cool 3410 |  | 2000019870 | Ondeo Nalco Co. |
| Tech Cool 3715 (350 gallon tote) |  | 1200181320 | Chemical Technologies |
| Tech Cool 3715B |  | 1200181400 | Chemical Technologies Inc. |
| Tech Cool 3718 |  | 2000011347 | CTI/Nalco Chemical co. |
| Tech Cool 5675 |  | 2000027195 | Ondeo Nalco Co. |
| Tech Cool 5907 EUO |  | 2000019855 | Ondeo Nalco Co. Inc. |

Appendix “C”

Recommended Machine Spare Parts List

Machine Type:

Machine Brass Tag Number:

| OEM Part Number | Part Description | Distributor Supplier | Distributor Supplier Part Number | Qty per Mach. | Delivery Lead Time Days | Unit Price | Where Used On Machine | Consignment Available |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Appendix “D”

EQUIPMENT PURCHASE AGREEMENT

THIS EQUIPMENT PURCHASE AGREEMENT (this "Agreement") is made as of __________ 2____, by and between Briggs & Stratton Corporation, a Wisconsin Corporation (“B&S”), and ____________________________, a _____________________ corporation (the “COMPANY”).

1.	RECITALS

WHEREAS the COMPANY desires to sell to B&S and B&S desires to purchase from the COMPANY certain equipment as described herein;

NOW THEREFORE, in consideration of the premises and mutual covenants contained herein, the parties hereby agree as follows.

2.	AGREEMENT

COMPANY hereby agrees to design, manufacture and sell to B&S, and B&S hereby agrees to purchase from COMPANY, the equipment itemized on Schedule A attached hereto.  The items of equipment are to be provided in accordance with the terms and conditions of this Agreement.

3.	DEFINITIONS

As used in this Agreement, the following terms shall have definitions as follows:

A.	SPECIFICATIONS mean a detailed and exact statement of particulars describing materials, dimensions, and features for the equipment to be built, including date of specification issue, part number and part name as described in Schedule A. Company Wide Equipment Standards shall apply if selected in Schedule A and if selected, shall be included as Attachment B.

B.	EQUIPMENT means the items of equipment described in Schedule A, including all amendments as may be submitted by B&S and accepted by COMPANY pursuant to this Agreement.

C.	PURCHASE PRICE means the price of the Equipment as shown on Schedule A.

D.	DELIVERY DATE means the date that the EQUIPMENT shall be completely stowed on-board the carrier at the COMPANY’s facility, suitably packed and crated for transit by the transportation method being used as shown in Schedule A.

E.	ACCEPTANCE DATE means the date that the EQUIPMENT shall successfully complete the ACCEPTANCE TESTS.

F.	ACCEPTANCE TESTS mean the tests described in Paragraph 17. of this Agreement.

G.	WARRANTY PERIOD means the period which begins on the ACCEPTANCE DATE and ends 2 years later.

H.	DEFECTIVE EQUIPMENT REPORT (D.E.R.) means a document which B&S shall provide to COMPANY during the WARRANTY PERIOD to confirm notice of defects in the EQUIPMENT.

4.	PURCHASE ORDER

The EQUIPMENT described in this Agreement is itemized on B&S’s Purchase Order      .  All EQUIPMENT purchases hereunder shall be subject to B&S’s Standard Terms and Conditions set forth on the back of the Purchase Order to the extent that they are not inconsistent with the provisions of this Agreement.  In the event of any ambiguity or inconsistency, this Agreement shall control.  From time to time, pursuant to the terms of Section 6 below, B&S may submit its Purchase Change Order to COMPANY for its acceptance.  Each such Purchase Change Order, upon COMPANY’s acceptance, shall effectively amend and be made a part of the Purchase Order.

5.	SPECIFICATIONS

A.	MANUFACTURING CONFORMANCE

COMPANY shall design and manufacture the EQUIPMENT to conform to the requirements set forth in the SPECIFICATIONS contained in Schedule A, or as amended by B&S in accordance with Section 6. of this Agreement.

B.	INTENDED USE

It is understood by COMPANY that B&S intends to use the EQUIPMENT primarily for the purpose of      .

6.	CHANGES TO SPECIFICATIONS

A.	CHANGE REQUESTS

If B&S proposes changes to the SPECIFICATIONS, COMPANY will give each such proposal its most serious and prompt attention.  COMPANY will promptly respond by stating in writing what effect, if any, such changes will have on the PURCHASE PRICE, DELIVERY DATE, and Warranty or Patent Indemnification provisions of this Agreement.  In the absence of a written statement from COMPANY regarding warranty or patent indemnification, it shall be conclusively presumed that the warranty and patent indemnification provision of this Agreement shall apply without modification if B&S elects to amend the SPECIFICATIONS with the change pursuant to Paragraph B of this section.

B.	CHANGE ORDERS

If B&S elects to incorporate changes in the SPECIFICATIONS into this Agreement, it is understood that any such changes shall be made and shall be evidenced by a written signed Change Order.  The Change Order shall amend the SPECIFICATIONS appropriately to incorporate the change and acknowledge the effect, if any, of the change on the PURCHASE PRICE, DELIVERY DATE and Warranty or Patent Indemnification provisions of this Agreement.  Such Change Orders shall be signed by an authorized representative of B&S.  If the COMPANY would be unable to accept a Change Order as written, the COMPANY will notify B&S within 10 working days of its receipt of the Change Order.  COMPANY’s silence will be construed as acceptance of the Change Order.

C.	TECHNOLOGICAL DEVELOPMENTS

COMPANY shall promptly advise B&S of all reasonably available technological advances that become known to COMPANY over the course of performance of its obligations under this Agreement which may result in the EQUIPMENT having added value (i.e. better performance, design, material, longer useful life, etc.) to B&S.  If B&S elects to incorporate such advances into this Agreement, it shall do so pursuant to Paragraph B of this Section.

7.	PRICE

The total PURCHASE PRICE of the EQUIPMENT is $      as shown on Schedule A, which may be amended from time to time pursuant to Section 6., Paragraph B. of this Agreement.

8.	PAYMENT TERMS

A.	PAYMENT SCHEDULE

The PURCHASE PRICE shall be paid according to the progress payment schedule outlined on Schedule A, unless amended by B&S in accordance with Section 6., Paragraph B.  of this Agreement.

B.	30 DAYS FROM DUE DATE

The progress payments referenced in Paragraph A of this Section will be made by check within 30 days from B&S’s receipt of COMPANY’s invoice pursuant to Paragraph C of this Section, provided that COMPANY shall, to B&S’s reasonable satisfaction, have completed the payment contingency outlined on the progress payment schedule.  In the event of a delay in COMPANY’s completion of the payment contingency, B&S shall have the right to delay the appropriate progress payment commensurately.

C.	INVOICING

COMPANY shall mail or deliver all invoices submitted hereunder to:

Accounts Payable

Briggs & Stratton Corporation

PO Box 702

Milwaukee, WI 53201

Invoices shall reference the Purchase Order number specified in Section 4. of this Agreement.  If B&S’s Purchase Order contains more than one item, COMPANY’s invoice will itemize those items being invoiced.  COMPANY understands that its failure to follow this requirement may result in delayed payments.

9.	DELIVERY

A.	ON TIME DELIVERY

COMPANY acknowledges that time is of the essence for delivery of the EQUIPMENT.  Therefore, COMPANY agrees to ship the EQUIPMENT on the DELIVERY DATE.

B.	OPTION TO DELAY DELIVERY

B&S shall have the right, at no additional charge, to postpone the delivery of the EQUIPMENT or any component for a period of 30 days maximum.  In the event of such a postponement, B&S shall have the right to commensurately delay its progress payments due by virtue of the shipment of the EQUIPMENT.

C.	TRANSPORTATION

COMPANY shall ship the EQUIPMENT as stated in Schedule A, using the carrier specified by the B&S Traffic Department and in accordance with B&S’s shipping instructions.  COMPANY shall call B&S's Traffic Department for routing (414-259-5352).  If the designated carrier is unavailable without a delay in delivery, COMPANY may substitute a carrier of a similar type and reputation, provided that COMPANY shall have first used its best efforts to notify B&S of the substitution.

D.	DELIVERY DELAYS - PREMIUM TRANSPORTATION

If shipment of the EQUIPMENT is delayed less than 15 calendar days after the DELIVERY DATE for any reason other than Force Majeure, B&S shall have the option to expedite the shipment of the EQUIPMENT by employing a premium transportation mode such as, but not limited to, exclusive-use van or air freight.  COMPANY hereby agrees to bear such premium costs if B&S elects to use such premium transportation mode in an effort to recover the time lost due to COMPANY’s late shipment.  Furthermore, COMPANY will use its best efforts to assist B&S in the routing or rerouting of the shipment to reduce delivery delay.

E.	DELIVERY DELAYS - IRREPARABLE

If shipment of the EQUIPMENT is delayed for any reason other than Force Majeure in excess of 15 calendar days after the DELIVERY DATE, the delay shall be considered an irreparable delay and a default by COMPANY under this Agreement.  In the event of such a default, B&S shall have in addition to any other remedies, the option to cancel this Agreement pursuant to Section 22.  If B&S elects to excuse the delay, it shall do so by a notice which shall include the adjourned date of delivery and may deduct from payments due under this Agreement $      for each day that delivery is delayed beyond the DELIVERY DATE.  Such adjourned date shall effectively revise the DELIVERY DATE.

F.	DELIVERY NOTICE

COMPANY shall notify B&S by FAX of an impending shipment 7 calendar days prior to actual shipment.  On the day of actual shipment, COMPANY shall confirm shipment by FAX notification which shall include the carrier’s name, the way-bill number, number of pieces, weight (by piece and total), progressive bill number, destination, etc.

G.	PACKING, ETC.

COMPANY will ship the EQUIPMENT in a manner consistent with general industry practice for shipping these kinds of goods so as to prevent any deterioration in transit.  Should it be necessary to ship the EQUIPMENT in a disassembled state, COMPANY shall ship the EQUIPMENT in the largest units possible consistent with expedient transportation of the EQUIPMENT.

10.	TITLE & SECURITY INTEREST

A.	B&S’s INTEREST

To secure B&S’s progress payments prior to the shipment of the EQUIPMENT, title to and a first security interest in the EQUIPMENT (and in any work in progress and materials required for the execution of COMPANY’s obligations hereunder, and any work which COMPANY may subcontract in the support of the performance of its obligations hereunder) shall vest in B&S to the extent B&S has made progress payments hereunder.

B.	COMPANY’s INTEREST

After B&S takes possession of the EQUIPMENT, B&S shall hereby grant to COMPANY a purchase money security interest in the EQUIPMENT to secure the amount of any unpaid portion of the PURCHASE PRICE.

C.	SECURITY AGREEMENT

This Agreement shall constitute the security agreement required by the Uniform Commercial Code of the appropriate State.  Each party will execute promptly any financing statement required to perfect and protect the interests of the other as defined in this Section 10.

D.	LIENS & ENCUMBRANCES

Both parties agree to pay the bills of their respective suppliers promptly and to comply with reasonable requests for evidence of payment.  However, either party shall have the right to withhold payment to any of its suppliers who have furnished defective, substandard or incorrect materials or workmanship.  Each party agrees to protect the interest of the other as defined in this section by not permitting any attachments to the EQUIPMENT of liens, encumbrances or claims for labor or material and to protect and hold the other harmless from all such claims, liens and encumbrances growing out of the design, manufacture, assembly, transit and installation of the EQUIPMENT.

11.	WARRANTY

A.	PERFORMANCE WARRANTY

COMPANY warrants that the EQUIPMENT will perform in accordance with the performance requirements set forth in the SPECIFICATIONS and that the EQUIPMENT has been designed and manufactured so as to conform to the SPECIFICATIONS.

B.	WORKMANSHIP WARRANTY

COMPANY further warrants that the EQUIPMENT will be merchantable, of good material and workmanship, free from defects and will be fit and sufficient for the purpose intended.

C.	DEFECT CORRECTION - PARTS & LABOR

In the event of a breach of either of the warranties outlined in Paragraphs A. and B. of this Section during the WARRANTY PERIOD, COMPANY shall without undue delay repair, replace or modify the EQUIPMENT so as to correct the warranty breach.

D.	B&S MAY REPAIR

B&S shall have the right to effect repair of any and all defects, if COMPANY shall have previously authorized such action or, in B&S’s reasonable commercial judgment, be unable or unwilling to effect the repair.  If B&S elects to take this action, it shall be entitled to deduct from any amounts owing to COMPANY the direct and incidental cost incurred in remedying the breach of warranty.  If B&S makes such deduction costs, B&S will furnish its bills and other documentation as it may have in its possession on request.  B&S’s action to effect cure of any warranty breach shall not relieve COMPANY of any of its obligations hereunder or under the Uniform Commercial Code except to the extent that B&S’s repair effort shall have directly caused further defects in the EQUIPMENT.

E.	NOTICE OF DEFECTS

B&S shall give COMPANY prompt notice of all defects known to it, either orally to COMPANY’s on site representative or by telephone to COMPANY’s customer service representative.  In addition, B&S will tender its D.E.R. to COMPANY confirming notice of such defects.  Upon receipt of such oral or telephone notice, COMPANY shall promptly and without undue delay notify B&S of its intentions and preferences to effect repair of the EQUIPMENT.

F.	PATENT INFRINGEMENT WARRANTY

COMPANY warrants that none of the EQUIPMENT, the use thereof or any of the applications, processes or designs employed in the manufacture thereof infringes the valid claims of any letter patent, patent application, copyright, trade secret or any other property right of any third party.

G.	INDEMNITY

In the event of a breach or alleged breach of this warranty, COMPANY shall, at its own expense, defend any suit or proceeding brought against B&S and shall fully protect and indemnify B&S against any liability, cost, recovery or other expense in or resulting from such suit or claim of infringement.  B&S shall give prompt notice in writing of any notice or claim of such suit, and B&S agrees to cooperate with COMPANY to enable it to make such defense.

H.	REMEDIES

If in any such suit or proceeding the EQUIPMENT or any part thereof is held to constitute an infringement and the full or partial use of the EQUIPMENT is enjoined, COMPANY shall, at no cost to B&S, (i) obtain for B&S a license to use the EQUIPMENT or (ii) modify the EQUIPMENT so as to avoid the infringement without any degradation in performance.

If COMPANY cannot obtain such a license, or the EQUIPMENT cannot be so modified, or if as a result of any suit of infringement or alleged infringement COMPANY is prevented from delivering the EQUIPMENT or any part or component thereof, COMPANY shall promptly refund to B&S all money paid by B&S for the EQUIPMENT.  This refund shall not be considered B&S's exclusive remedy.

12.	DOCUMENTATION AND SPARE PARTS

A.	The drawings will include descriptions of all components associated with a particular system and contain an itemized table of all purchased parts and tooling including the name of the manufacturer and the manufacturer’s part number.  COMPANY shall supply B&S with full assembly, hydraulic, pneumatic, lubrication, electrical and tooling and shall also supply B&S with an Operator’s Manual and any other documentation related to the operation, maintenace or repair of the EQUIPMENT.

B.	COMPANY shall supply B&S with a recommended Spare Parts List for the EQUIPMENT 60 days prior to shipment.  Spare parts list shall include the usage, recommended safety stock, price, and delivery (reference Appendix C of Terms and Conditions of Equipment Purchase)  COMPANY agrees for a period of 5 years after expiration of the WARRANTY PERIOD to sell to B&S at prevailing delivery and payment terms, all necessary spare parts required for maintenance of the EQUIPMENT.  Spare parts will be at catalog prices.  Further, COMPANY agrees within the limits of production capability to provide emergency spare parts within a 24 hour period after receipt of the emergency order.  COMPANY shall provide B&S with immediate access to drawings so spare parts can be machined by or on behalf of B&S in the event of an emergency and the spare parts are not in stock for immediate use by B&S.  If COMPANY ceases to support the EQUIPMENT as stated in the preceding paragraph, COMPANY agrees to supply B&S with all necessary documents including manufacturing drawings so that B&S can maintain the EQUIPMENT.

C.	COMPANY shall make available to B&S, twenty-four (24) hours a day and seven (7) days a week, technical and spare parts support.  The COMPANY shall provide the support through sufficiently competent, skilled and qualified personnel that can expedite spare parts, provide technical assistance and dispatch service personnel as required by B&S.  The COMPANY shall complete Company Profile Sheet for personal contacts (referenced in Company Wide Equipment Standards, Section XV)

13.	DOCUMENTATION INSTRUCTIONS

COMPANY shall place software, sources, codes and related documentation in escrow for a period of 7 years after expiration of the WARRANTY PERIOD to be delivered to B&S (A) in the event that COMPANY shall cease conducting business in the normal course, or shall become insolvent, or make a general assignment for the benefit of creditors, or suffer or permit the appointment of a receiver of its business or assets, or shall avail itself of or become subject to any proceeding under any bankruptcy act (domestic or foreign), or any state relating to insolvency of the protection of the rights of creditors, or (B) in the event COMPANY shall withdraw any of the products or otherwise cease supporting the products.

14.	RISK OF LOSS

COMPANY shall bear risk of loss until the EQUIPMENT shall come to rest aboard the motor carrier for shipment to B&S.  After timely notice under Section 9.G., B&S shall bear risk of loss.

15.	INSURANCE

A.	PRIOR TO SHIPMENT

While COMPANY has risk of loss, COMPANY agrees at its own expense to procure and carry suitable fire, sprinkler leakage and extended coverage insurance on material, work-in-process and any furnished items which comprise or will comprise the EQUIPMENT.  The amount to be insured shall be the actual replacement value of said material, work-in-process and furnished items.  Such insurance shall provide a loss payable clause in favor of B&S as its interest may appear.

B.	AFTER SHIPMENT

While B&S has risk of loss, B&S agrees to procure and carry as appropriate in-transit, sprinkler leakage, fire and extended coverage on the EQUIPMENT covering the full insurable value of the EQUIPMENT.  Such insurance shall provide a loss payable clause in favor of COMPANY as its interest may appear.

C.	DURING INSTALLATION

If performance of COMPANY’s obligations hereunder requires COMPANY’s employees or agents to work on B&S’s premises, COMPANY shall secure at its own expense and prior to such activity:

(i)	Workmen’s Compensation and Occupational Disease Insurance as required by law.

(ii)	Comprehensive Public Liability Insurance in limits of not less than $300,000 per person and $1,000,000 in aggregate on all claims.

(iii)	Liability insurance against damage to property of not less than $500,000.

D.	EXCESS AND PRODUCT LIABILITY

COMPANY shall purchase and maintain excess and product liability insurance from an independent insurance company with respect to the EQUIPMENT sold to B&S.  The policy shall have coverage limits of at least $5 million per claim and name B&S as an additional insured.

E.	CERTIFICATES

All insurance maintained by either party shall be in companies acceptable to the other.  Upon written notice by the other, each party shall provide certificates of insurance evidencing such insurance signed by an authorized representative of the insurance company.  In the event of any material change in or cancellation of the coverage, at least 10 days prior written notice will be given to B&S.

16.	TAXES

B&S shall reimburse COMPANY for any sales tax, use tax or any other federal, state or local tax which is measured solely by the PURCHASE PRICE and required to be paid by COMPANY by virtue of the sale and delivery of the EQUIPMENT.  In case of doubt by B&S as to COMPANY’s liability for any such tax, COMPANY shall allow B&S, at its own expense, to assume control of any litigation or proceeding relating to the determination and settlement of such tax.  B&S shall, upon final settlement of such litigation and proceeding, reimburse COMPANY for any tax owing including in the amount to be reimbursed any interest charges and penalties accruing thereon.  Should B&S submit with this Agreement a Machinery and Equipment Exemption Certificate or similar document, COMPANY agrees to cooperate with requirements outlined therein.

17.	ACCEPTANCE/PERFORMANCE TESTING

A.	ACCEPTANCE TEST AT COMPANY

Prior to shipment, B&S shall be entitled to conduct a preshipment ACCEPTANCE TEST pursuant to the procedure outlined in Schedule A.  The purpose of this test shall be primarily to identify any mechanical interferences between machine members, improper adjustments made during assembly, and generally to determine that the EQUIPMENT is in good working order.  COMPANY may observe said preshipment testing.  It is understood that all defects in materials and workmanship that may be identified as a result of this testing will be corrected by COMPANY prior to shipment of the EQUIPMENT unless specifically directed by B&S to the contrary.

B.	ACCEPTANCE TEST AT B&S

Upon completion of installation and adjustment of the EQUIPMENT, COMPANY’s installation supervisor shall notify B&S’s on-site representative that the EQUIPMENT is installed and ready for ACCEPTANCE TESTING.  B&S shall promptly commence the ACCEPTANCE TEST procedure assisted by COMPANY (if COMPANY chooses to assist).  The ACCEPTANCE TEST procedure shall be conducted pursuant to the procedure outlined in Schedule A.

C.	CRITERIA FOR ACCEPTANCE

On the ACCEPTANCE DATE, B&S shall certify to COMPANY, in writing upon request, of its acceptance of the EQUIPMENT, provided that the EQUIPMENT shall have demonstrated during the ACCEPTANCE TEST its performance to be in accordance with the SPECIFICATIONS.  The acceptance of the EQUIPMENT shall in no way release COMPANY of any of its Warranty or any other obligations hereunder.

D.	PROVISIONAL ACCEPTANCE

In the event that the ACCEPTANCE TEST identifies areas of performance non-conformance which in B&S’s unilateral commercial judgment do not materially impair B&S’s use of the EQUIPMENT in the short term, B&S shall have the option to certify its provisional acceptance of the EQUIPMENT.  In this case, B&S shall itemize those areas in which the EQUIPMENT has failed to perform acceptably.  COMPANY shall acknowledge said list and inform B&S, in writing, as to when the non-conformance shall be corrected.  COMPANY agrees that time is of the essence with regard to such corrections.  B&S shall have the right to withhold payment of any funds normally due by virtue of acceptance.  The amount of the payment withheld shall be determined by B&S and shall be reasonably commensurate with the reduced value of the EQUIPMENT.  Upon completion of the modifications by COMPANY, it shall notify B&S of same and the ACCEPTANCE TEST shall be rerun in its entirety should B&S elect to do so.  Should COMPANY fail to meet the corrective modification completion date, B&S may, at its option, revoke its provisional acceptance and use any of its remedies as may be provided herein or by the appropriate Uniform Commercial Code.

18.	REVIEWS

A.	PRIOR TO MANUFACTURE

COMPANY shall review the design of the EQUIPMENT on B&S’s site prior to execution of this Agreement.  B&S shall have the option to review the design again on its site during the design period to verify COMPANY progress and compliance with the SPECIFICATIONS.  Prior to the release of the design to manufacturing, COMPANY shall host a final design review on its own site.  It is understood that all designs, layout and detail drawings, etc. necessary for B&S’s verification and review will be made available during these reviews.

B.	PRODUCTION SCHEDULE

To enable B&S to evaluate COMPANY’s progress in the design and manufacture of the EQUIPMENT, COMPANY, within 30 days after execution of this Agreement, shall supply B&S with a predetermined production schedule, indicating the planned duration of the manufacturing and erection phases of the EQUIPMENT and at B&S’s request will report on the first day of each month to B&S as to progress.  This report will be given in such a manner as to enable B&S to evaluate the progress being made by COMPANY in the major production steps as against the predetermined schedule.  Within 30 days from acceptance of any Purchase Change Order submitted pursuant to this Agreement, COMPANY shall also submit for review by B&S, a schedule of production dates of the EQUIPMENT as called for in the Change Order Specifications.

C.	DURING MANUFACTURE

During the period COMPANY is manufacturing the EQUIPMENT, B&S shall have the right of access to any of COMPANY’s plants as may be necessary to allow B&S to review COMPANY’s progress, verify that the EQUIPMENT is being manufactured so as to conform to the SPECIFICATIONS, and verify that a progress payment contingency has been completed.  To expedite these reviews and maintain some continuity, the parties shall each nominate one person to act as their companies’ respective coordinators for information flow between companies.  It is understood that these coordinators shall be responsible for project logistical information only.  All notices of legal or commercial importance shall be made pursuant to Section 24 herein.

19.	INSTALLATION

A.  COMPANY shall provide the services of an experienced, qualified individual to supervise the installation.  COMPANY shall acknowledge receipt of B&S’s Environmental, Health and Safety Manual for Contractors dated November 2003 (or as revised) and agrees to conduct all activities in conformance with that Manual (Attachment C).  The installation supervisor shall be responsible for verifying that the EQUIPMENT has been properly unpacked and installed including verification of electrical systems, fluid levels and motor rotation.  The COMPANY’s installation supervisor is responsible for the initial power up and cycling of the EQUIPMENT and shall provide instruction to B&S on operation of the EQUIPMENT.  Upon completion of installation, the COMPANY’s installation supervisor shall notify B&S’s on-site representative that the EQUIPMENT is installed and ready for the ACCEPTANCE TEST.  COMPANY may, at its option and expense, elect to participate in the ACCEPTANCE TEST.

B.	INSTALLATION CHARGES

The cost for installation, not to exceed      , shall be billed at COMPANY’s normal standard hourly rate for such services which is currently in effect at the time of installation.  Notwithstanding any of COMPANY’s policies to the contrary, overtime and weekend work shall be billed according to the following schedule:

DESCRIPTION		% OF STD. RATE

Hours over 8 hours Mon. - Fri.		150%

Saturday			150%

Sunday			200%

Overtime and weekend work premiums shall only be chargeable when expressly authorized by B&S’s Representative.  Overtime and weekend work premiums will also not be chargeable when such work is required due to COMPANY’s late delivery of EQUIPMENT.

Time spent during installation to correct defects and warranty repairs shall not be chargeable.  COMPANY agrees that the price itemized on Schedule A for installation is the maximum chargeable for the work exclusive of expenses.

Time spent commuting to and from the B&S’s facility shall not be billable.

C.	LIVING & TRAVEL EXPENSE

B&S shall reimburse COMPANY pursuant to invoice for the installer’s living and travel expenses which can be shown to be incurred directly as a result of the installation of the EQUIPMENT.  Unless otherwise approved in writing, travel expenses shall not exceed one round trip fare including air and/or surface transportation from the installer’s domicile to B&S’s installation site.  All reimbursements shall be made at the actual amount spent as documented by expense receipt.  Living arrangements used shall be commensurate with the arrangements made by B&S for its own personnel staying in the same geographic area.  All air travel shall be coach class.

D.	INSTALLATION SCHEDULE

B&S shall provide COMPANY access to the EQUIPMENT during normal working hours.  Should the installation of the EQUIPMENT require the curtailment of any operational production equipment, COMPANY agrees to coordinate the installation so as to minimize the effect of the required downtime on B&S’s production. B&S agrees to use its best efforts to schedule the production equipment so as to permit the installation to be carried out as soon as possible after delivery.

E.	TOOLS AND SHOP FACILITIES

COMPANY shall provide any specialty tools and materials necessary for normal installation of the EQUIPMENT.  B&S’s maintenance personnel shall cooperate with COMPANY’s installation supervisor to the extent possible.  However, COMPANY understands that B&S’s shop facilities are intended for the maintenance of a production facility with critical production timing requirements.

F.	RULES OF WORK

COMPANY agrees that, while its personnel are on B&S premises, they will abide by B&S’s normal rules of work and its standard practice governing behavior of its own employees.  Also, COMPANY agrees to require such personnel to work in a manner which will comply with the Occupational Safety and Health Act of 1970.

20.	COMPLIANCE WITH LAWS

A.	PURCHASE ORDER CONDITIONS

The provisions contained in the Governmental Compliance section of B&S's Standard Conditions of Purchase are hereby restated into this Agreement by reference.

B.	OSHA - COMPANY AS AN EMPLOYER

The EQUIPMENT will be manufactured in a manufacturing environment which will comply with the provisions of the Occupational Safety and Health Act of 1970, all similar state and local acts, and any standards and regulations issued thereunder.

COMPANY agrees that any failure to comply which would result in a delay in COMPANY’s performance of its obligations hereunder shall not in any way excuse COMPANY from the performance of its obligations under this Agreement.

C.	OSHA - COMPANY AS MANUFACTURER

COMPANY certifies that the EQUIPMENT shall be designed, manufactured and assembled so as to comply with the Occupational Safety and Health Act of 1970 and any and all Federal, state and local acts, regulations or codes to the extent that the provisions and standards set forth therein apply to the EQUIPMENT.  This compliance shall be limited to all standards, regulations issued and/or interpretations (in citations or judgments made) prior to the DELIVERY DATE.  B&S shall assist COMPANY by using its best efforts to notify COMPANY of any such standards, regulations and interpretations as may become known to B&S during the manufacture of the EQUIPMENT.

D.	OSHA - INDEMNITY

COMPANY agrees to indemnify and hold B&S harmless for any loss, damage, fine or penalty which may be due to a violation of such acts, codes, etc. as defined in this Section.  This indemnity is subject to and conditioned upon B&S’s use of the EQUIPMENT for the purpose for which it was intended.  It shall not apply to any violations, damages, or losses resulting in whole or in part from modifications made by B&S subsequent to installation and acceptance of the EQUIPMENT.

E.	OSHA - COMPANY IS DESIGNER

COMPANY agrees that for so long as B&S shall own and use the EQUIPMENT, COMPANY shall use its best efforts to assist B&S in the redesign and modification of the EQUIPMENT should such action be necessary as a result of a code or health act violation or citation.  COMPANY agrees to so assist on an emergency basis should B&S be enjoined from using the EQUIPMENT as a result of such violation or citation.

It is understood that such redesign and modification effort shall be paid for by B&S based on COMPANY’s actual costs to do the work and a reasonable premium for emergency basis assistance should it be necessary.

F.	PARTIAL INVALIDITY

The parties agree that they will perform their obligations hereunder in accordance with all applicable laws, rules and regulations now and hereafter in effect.  If any term or provision of this Agreement shall be found to be illegal or unenforceable, this Agreement shall remain in full force and effect and such term or provision shall be deemed stricken and replaced with provisions that preserve the same economic effect.

21.	TERMINATION

Without limiting any rights or remedies which B&S may have in the event of any default or failure of performance by COMPANY, B&S shall have the right, upon 10 days prior written notice to COMPANY, to terminate this Agreement at any time and without cause prior to complete delivery.

Any such termination shall be without any obligation or liability to COMPANY other than payment of charges for the value of work performed, and for necessary expenditures which can be established by COMPANY as having been reasonably incurred prior to the time that notice of termination is given.

In no event shall the termination charges exceed the PURCHASE PRICE.

Any progress payments of the PURCHASE PRICE shall be credited against the amount of such charges, and in the event the payments exceed the charges the excess shall be returned to B&S.  In the event of any termination, B&S shall be entitled to all materials, work in process and completed work included as value of work performed and necessary expenditures in determining the charges referred to above and paid by B&S.

22.	CANCELLATION

If COMPANY shall fail to comply with the SPECIFICATIONS or drawings in any material respect, or if it shall fail substantially to comply with any other provision of this Agreement, then B&S may, without prejudice to any other right or remedy and upon giving 10 days prior written notice, cancel this Agreement in whole or in part and/or reject all or part of the EQUIPMENT herein whether or not the manufacture thereof has been completed or such machinery has been received at the plant of installation.

Upon notice of cancellation by B&S, COMPANY will promptly return all moneys previously paid to COMPANY by B&S for the EQUIPMENT (if any) and direct damages equal to:

A)	all costs incurred by B&S in preparation for the EQUIPMENT including but not limited to training, systems programming, site preparation, cost of supplies and facilities excepting only such costs which can be applied to EQUIPMENT obtained from any other manufacturer, and

- the difference in price between the EQUIPMENT to be delivered hereunder and the equivalent substitute equipment.  In no event shall COMPANY be liable for any losses of anticipated profits or other consequential damages.

23.	BANKRUPTCY

If COMPANY shall be adjudicated bankrupt, or if it should make a general assignment for the benefit of creditors, or if a receiver shall be appointed due to its insolvency, B&S may without prejudice to any other right of remedy terminate this Agreement and at its option may take possession of the EQUIPMENT being manufactured under this Agreement to indicate ownership by B&S.

To the extent reasonably possible, materials and work in process pertaining to the EQUIPMENT shall be stored separately from other stock and marked conspicuously with labels indicating ownership by B&S.

24.	NOTICES

All notices hereunder shall be in writing.  These notices shall be deemed duly given upon delivery if delivered by hand (against receipt) or 3 days after posting, if sent registered mail, return receipt requested.

In the case of notices given to B&S: 	In case of notices given to COMPANY:

Briggs & Stratton Corporation

12301 West Wirth Street

Wauwatosa, WI 53222

Attention:      	Attention:

e-mail:       	e-mail:

Fax:      	Fax:

Or to whomever else the parties may designate by notice pursuant to this Section.

25.	GENERAL TERMS

A.	PROMOTION LIMITATION

COMPANY agrees that it will not use B&S’s name, whether by including reference to B&S in any list of customers advertising that its services or products are used by B&S or otherwise, without written authorization by B&S’s authorized representative.

B.	SURVIVAL BEYOND COMPLETION

The provisions of this Agreement shall survive the delivery of the EQUIPMENT and the payment of the PURCHASE PRICE.

C.	AMENDMENTS

No amendment to this Agreement shall be effective unless it is in writing and signed by duly authorized representatives of both parties.

D.	COMPLETE AGREEMENT

This Agreement represents the entire Agreement between the parties with respect to the purchase and sale of the EQUIPMENT.  All prior agreements, representations, statements, negotiations and undertakings whether oral or written are deemed incorporated herein.

E.	EFFECT OF WAIVER

No provision of this Agreement shall be deemed waived and no breach excused, unless such waiver of consent shall be in writing and signed by the party claimed to have waived or consented.  Any consent by any party to, or waiver of, a breach by the other, whether express or implied, shall not constitute a consent to, waiver of, or excuse of any other different or subsequent breach.

F.	MOST FAVORED CUSTOMER

All of the prices, terms, warranties and benefits granted by COMPANY herein are comparable to or better than the equivalent terms being offered by the COMPANY to any present customer.  If COMPANY shall, during the term of this Agreement, enter into arrangements with any other customer providing greater benefits or more favorable terms, this Agreement shall thereupon be deemed amended to provide same to B&S.

G.	AUTHORITY

Each party represents that it has full power and authority to enter into and perform this Agreement, and the person signing this Agreement on behalf of each has been properly authorized and empowered to enter into this Agreement, understands it, and agrees to be bound by it.

H.	HEADINGS NOT CONTROLLING

The headings used in this Agreement are for reference purposes only and shall not be deemed a part of this Agreement.

I.	APPLICABLE LAW

This Agreement will be governed by and construed in accordance with the internal laws of the State of Wisconsin without regard to its conflicts of law principles.

J.	FORCE MAJEURE

Neither party shall be liable for a delay in its performance of its obligations and responsibilities under this Agreement due to causes beyond its control, such as but not limited to war, strikes or lockouts, embargo, national emergency, insurrection or riot, acts of the public enemy, fire, flood, or other natural disaster, provided that such party has taken reasonable measures to notify the other in writing of the delay.  Failure of subcontractors and inability to obtain materials shall not be considered as a Force Majeure delay.

K.	ADDITIONAL WARRANTIES AND INDEMNITIES

COMPANY agrees to indemnify and hold harmless B&S against all claims, losses, damages, attorney fees and costs arising from personal injury, death or property damage which occurs as a result of any act or omission by the COMPANY or its agents or contractors while on B&S's premises, except to the extent that B&S is determined to have been negligent in causing such injury, death or property damage.

COMPANY agrees to indemnify and hold harmless B&S and its directors, officers, employees, agents, distributors, dealers and resale customers with respect to all claims, losses, damages, attorney fees and costs arising from personal injury, death or property damage claimed to result directly or indirectly from the EQUIPMENT sold to B&S under this Agreement and resulting from any breach of this Agreement or any negligent act or omission by the COMPANY.

L.	ASSIGNMENT

COMPANY shall not assign this Agreement or its obligations hereunder to any third party without prior written consent from B&S.  B&S shall not assign this Agreement or its obligations hereunder to any third party except its subsidiaries without prior written consent from COMPANY.  Neither consent will be unreasonably withheld.

[NAME OF COMPANY]	BRIGGS & STRATTON CORPORATION

By  ___________________________________	By _________________________________

Name:      	Name:

Title:      		Title:

SCHEDULE A

EQUIPMENT PURCHASE AGREEMENT

(Rev. 8-05)

Par. 3B)  Equipment Description

Par. 5)  Specifications

Par. 3A)  Yes    No  Company-Wide Equipment Standards (Rev. August 1, 2005) apply.  If "yes" they are attached as Attachment "B"

Par. 7)  Purchase Price

Par. 9)  Delivery Date, Method and Location

Delivery Date:        ,

Method:

Ship to Location :      FOB:

Par. 8A)  Progress Payment Schedule

Par. 17)  Acceptance Test

Procedure at Company:

Procedure at Briggs & Stratton Corp.:

APPENDIX “E”

VENDOR INSTRUCTIONS FOR DRAWINGS

PURPOSE:	This document provides instructions for the creation of proper documentation of special machines, equipment, tooling, fixtures or dies for Briggs and Stratton.

SCOPE:	This applies to all tooling, fixturing, dies (die cast and stamping) and machines / equipment that is designed for Briggs and Stratton by outside sources for manufacturing purposes.  This information is necessary to ensure that documentation received from the Company complies with the Briggs and Stratton Tool Drawing Management System.

TOOL DESIGN & FIXTURE REQUIREMENTS

General

At the time of delivery of special machines, equipment, tooling, fixtures or dies to Briggs & Stratton, the Company shall provide Briggs & Stratton with a complete set of drawings including all assembly drawings with a corresponding Bill of Material (B.O.M.), all manufactured details completely dimensioned, all altered purchased items with alterations completely dimensioned, all special cutting tools, all multi-spindle/cutter heads and all tool layout drawings.

The Company shall satisfy all of Briggs & Stratton’s documentation requirements as specified within this document.  The Company will not be considered to have fully performed under any purchase order for the special machines, equipment, tooling, fixtures or dies until the Company satisfies Briggs & Stratton’s documentation requirements.

Briggs & Stratton Manufacturing Engineers/Tool Designers will provide tool numbers at the time of purchase agreement (concept approval of engineering drawings).

All mechanical, electrical, hydraulic, pneumatic and other drawings shall be prepared using Briggs & Stratton format.  All drawings will be CAD generated, in full-scale geometry and scaled during printing, with the scale in title block reflecting printing scale.  No pencil drawings will be accepted.  Briggs & Stratton will provide the CAD drawing border formats.  All detail sheets will be the same size as the main assembly or maximum of actual “E” size and contain multiple detail.  Do not explode dimensions or blocks.

Standard drawing form sizes:

“A” Size  -  B&S Size “2”  -   8.5” x 11”

“B” Size  -  B&S Size “3”  -   11” x 17”

“C” Size  -  B&S Size “4”  -   17” x 24”

“D” Size  -  B&S Size “5”  -   24” x 36”

“E” Size  -  B&S Size “6”  -   36” x 48”

Title Block:  Final drawings SHALL have a Briggs & Stratton title block completed in the lower right hand corner.  A Briggs & Stratton title block minimally consists of the name of the tool, tool and operation number, printing scale, any special tolerances, number of sheet and total number of sheets and date of drawing.  Drawing borders as supplied to the Company will be minimally defined (brass tag, dept, etc).

Detail Numbering:  On assembly drawings, the Company shall use split balloon for detail numbers.  Balloons shall be 5/8” in diameter, with the upper half reflecting the detail number and the lower half reflecting the drawing number (ie.  T10542-010-010).  On detail sheets, the Company shall balloon all details with detail number, quantity required, material type, if hardened, give Rockwell “C” reading, and if detailed is to be ground.

Drawing changes:  if any changes are made to drawings after the fabrication of the special machines, equipment, tooling, fixtures or dies, the change block in the main title block shall be filled out with the corresponding change.  The Company shall provide Briggs & Stratton with updated documentation immediately upon the implementation of any change, whether the change occurs before or after installation.

Dimensioning:  In general, it is desirable to have assemblies dimensioned for ease of programming CNC machines and trouble shooting equipment.  Briggs & Stratton uses the practice of replacement details on fixtures.  Detail dimensioning shall be done accordingly.  Good dimension placement and practice shall be observed.  Dimensions shall be in decimal inch format.

Marking:  All cutting tools, fixtures and special machines shall be permanently marked with its assigned tool number and corresponding detail number.

C.N.C. machining fixtures to have zero indicating holes when needed.  All PSI settings and sequence of operations shall be reflected on the assembly drawings.

If the Company is involved in special machine tool layouts, the Company shall include the following information in its quotation:

Cutting Speed (RPM and SFM)

Feet per Revolution

Feed Rate (inch per minute)

Rapid Advance (time and distance)

Dwell Time

Total Feed

Total Stroke

Gage Line Set-Up for Length of Tool

Description of Tool (i.e., Drill, Reamer, Finish Bore, etc.)

Consideration shall be given to special machines, cutter heads and tool designs to allow adequate room for tool change.  All tools shall be removable with part in fixture.

All tooling shall have pre-set capabilities when ever possible.

All cutting tool assemblies (boring bars, tool holders, etc.) shall be individually detailed on Briggs & Stratton media.

Briggs & Stratton shall approve special or combination tools.

The Company shall consider standard tooling as the first option.

DOCUMENTATION REQUIRED

# Machine Documentation

All prints and BOM shall be by B&S CAD readable format.

All prints shall be on Briggs & Stratton borders.

All prints shall be in English.

Tooling prints in decimal inch format.

All prints refer to Briggs & Stratton Brass Tag number.

Manuals in English with the following sections as applicable:

Preventative Maintenance section.

Error message and corrective action.

Descriptive machine functions.

Control panel layout with button descriptions.

Cycle start up procedure.

Shut down procedure.

Power up procedure.

E-stop recovery.

Axis reference procedure.

Manuals should be provided in electronic format.

# General Documentation Required

Machine assembly

Machine floor plan in CAD format

Fixtures

Special tools and tool layouts

Special work heads

Electrical

Hydraulic

Pneumatic

Coolant/Lubrication needed

Spare Parts List

Purchased Components – catalog numbers and operating instructions

# Base Machine Final Documentation

The Company shall supply complete documentation as listed below before equipment ACCEPTANCE and payment.

The Company shall supply Briggs & Stratton with a complete set of original detail drawings for all specially designed equipment within 30 days of ACCEPTANCE of equipment.  All drawings shall be in electronic format, shall use Briggs & Stratton borders and be supplied on computer disk in AutoCAD dwg format.  (Special requests for dxf or IGES or 3D CAD formats will be considered.) All drawings shall adhere to Briggs & Stratton "Tool Design Drawing Standards" and use the Briggs & Stratton drawing-numbering system with a set of drawing numbers supplied by the Briggs & Stratton Project Leader.

The Company is to supply Briggs & Stratton three (3) hard copies of operating and maintenance manuals as well as a copy in electronic format (check with Project Leader for acceptable file types).  These manuals shall include the following information:

Company's name, telephone number and project identification number.

Basic equipment start-up and operating instructions.

Recommended preventative maintenance program.

A complete list of consumable items (filters, tools, oils, etc.)

Maintenance instructions.

A complete parts list with all purchased parts identified by the original manufacturers name and part number.

Name and telephone number of any company supplying major components. (Spindles, slides, coolant system, etc.)

Recommended lubricants .

All available manufacturers manuals for purchased components.

The Company is to provide a recommended spare parts list.  This list shall be dated and include price and delivery for each item.

# Fixture, Tool and Accessory Final Documentation

In addition to the documentation listed above, the documentation described below shall be submitted to the Briggs & Stratton Manufacturing Engineer.  All documentation shall be in English (Decimal Inch) and comply with Briggs & Stratton standards.

One (1) hard copy to scale for each drawing.

Files shall contain only one drawing per file name.

Files shall have names that correspond to Briggs & Stratton tool numbers.

Electronic documents/files shall be transmitted via one of the following media:

CD – ROM  (preferred)

FTP – Briggs & Stratton Website

DRAWING NUMBERING SYSTEM INSTRUCTIONS

The tool design /drafting group(s) in the manufacturing area coordinates the assignment of all drawing numbers.  Requests for drawing numbers assignment or changes should be made through them.  Drawing numbers will be formatted in the following manner:

T10XXX    -    000    -    001

Parent Number				Drawing Number

## Group Number

Parent Number - 	This portion of the number is pre-assigned and will be unique to each new machine or fixture.  This number will be referenced in the header of the Tooling T-Sheet (see sample T-Sheet below).

Group Number - 		This portion of the number will be used to help identify different categories for the new machine or fixture.  Simple machines, fixtures or tools will use a “-000” group number, while more complicated machines, such as dials, transfer lines etc, will use the pre-defined group numbers as listed below.

Drawing Number - 	This portion of the number records the number of the drawings in the “group” and will be sequential numbers beginning with “-001”.

# Group Numbering Naming Structure

T10XXX	Parent Number

T10XXX - 000	Simple Machine / Tools / Fixtures

T10XXX - 010	Base Machine (Plan Views, Elevations, Layouts, etc.)

T10XXX - 020	Electrical / Controls

T10XXX - 030	Hydraulic

T10XXX - 040	Pneumatic

T10XXX - 050	Coolant

T10XXX - 060	Lubrication

T10XXX - 070	Fixtures

T10XXX - 080	Guarding

T10XXX - 090	Tooling

T10XXX - 100	User Defined

T10XXX - 110	User Defined

T10XXX - XXX	The designer or engineer would determine how any additional “User Defined” group number(s) would be applied depending on the specific project.  This procedure applies to both internal and vendor based Tooling projects.

SPREADSHEET (T- SHEET) SYSTEM

A Microsoft Excel spreadsheet will be provided by the tool design / drafting group and will be used to document the new machine, fixture, tool or die drawing numbers and descriptions.  The header part of the spreadsheet will be filled out (see example) and tabs (worksheets) will be used to separate each of the groups.  The cover sheet will be used to identify all of the subgroups that will be used to document the new machine or fixture.  Placing an “X” next to the corresponding group category will identify the drawing groups considered to be active.

It’s the responsibility of the Company to provide any and all new tabs (worksheets) as required to document the new machine or fixture.  The Company is also responsible for filling in the information beginning in part B below.

In general, for Simple Machines, the use of the T-Sheet is optional.  Please note that at a minimum, the cover sheet and the 000 group worksheet tab should be filled out if a spreadsheet is used to document the machine or fixture.  However, for Complex Machines (large fixtures, transfer lines, multi-station dials etc), the use of the T-Sheet is required.

The following data fields are to be filled out by the tool design / drafting group:

(1)  “T” Number  -  The parent number as assigned for the new machine or fixture.

(2)  Machine Description  -  The verbal description for the specific machine or fixture.

(3)  Type  -  This refers to the type of machine or fixture described (Die Cast, Stamping, Cold Heading, Assembly etc.)

(4)  Plant  -  This field identifies the final plant location for the machine or fixture.

(5)  Brass Tag #  -  This refers to the accounting number for the capital asset.

(6)  Dept #  -  This refers to the manufacturing area inside the plant where the machine or fixture will be located.

(7)  Mod #  -  This reference field identifies the specific engine model number that the finished production parts made on the machine or fixture will be used.

(1)			(2)				(3)	(4)	(5)	(6)	(7)

The following data fields are to be filled out by the tool design Company:

- Active Group number  -  This field on the main cover sheet indicates which of the remaining tabs are activated and will contain drawing information.  A letter “X” should be used to identify active spreadsheet tabs.  Those tabs that are not used can be left blank or marked with “NOT USED” (see example 2).  If additional groups are required to further define the machine or fixture, they should be added and described beginning with 100, continuing to increment by 10 (110, 120, 130 etc).  Corresponding tabs (worksheets) should also be added to the spreadsheet and properly filled out accordingly.

(1)

Tabs (worksheets)

(2)  Sheet #  -  This field will record the actual drawing file name and should be filled out using the full T10XXX-XXX-XXX series formatted number.

(3)  Letter Description  -  This field holds the description of the drawing file.

(4)  Vendor Number  -  This field holds the equivalent Company drawing number.

(5)  Name  -  This filed holds the initials of the person who created the drawing (optional).

(6)  Date  -  This field holds the date that the drawing was created (optional).

2)		(3)				(4)		(5)		(6)

Drawing Border Format

Partially filled out AutoCAD drawing borders will be provided by the tool design / drafting group and will be used to document the new machine or fixture.  Fields to be filled out on the blank drawing borders as provided to the Company include:

(1)  Asset #

(2)  Title  (this is a partial description to be expanded on)

(3)  Dept No.

(4)  Mod. #

(5)  Design Vendor

(6)  Drawing Number  (this is a partial formatted number).

Fields to be filled out by the Company include:

- Drawn

- Date

- Scale

- For Assy. See Dwg. #

(1)		(2)		(3)		(4)		(5)		(6)

(7)		(8)		(9)		(10)

Recommended Machine Spare Parts List

(Also in Appendix C of Company wide Equipment Standards)

Machine Type:

Machine Brass Tag Number:

| OEM Part Number | Part Description | Distributor Supplier | Distributor Supplier Part Number | Qty | Lead Time | Unit Price | Where Used On Machine | Consignment Available |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | ##### APPENDIX “F” ##### REVISION LEVEL DETAILS |
| --- | --- |
| REVISION DATE | REASON FOR REVISION |
| 07/21/05 | The Equipment Specification rewritten with new format |